Milesight Network IP Camera Application Programming Interface.

CGI Specification

Version 1.4

2015.12.18

Table of Contain

Revision history	5

Overview	5

Permission level	5

Valid values	6

1. admin.cgi	6

1.1 NETWORK	7

1.1.1 get.network.general	8

1.1.2 set.network.general	9

1.1.3 get.network.advanced	10

1.1.4 set.network.advanced	11

1.1.5 get.network.ddns	11

1.1.6 set.network.ddns	11

1.1.9 get.network.wireless	13

1.1.10 set.network.wireless	16

1.1.11 search.network.wireless	16

1.2 SYSTEM	17

1.2.1 get.system.information	17

1.2.2 set.system.information	17

1.2.3 get.system.datetime	18

1.2.4 set.system.datetime	19

1.2.5 get.system.maintenance	20

1.2.6 reboot.system.maintenance	20

1.2.7 loaddefault.system.maintenance	20

1.2.8 backupfile.system.maintenance (not implemented)	20

1.3 USER	21

1.3.1 get.system.user	21

1.3.2 set.system.user.anonymous	21

1.3.3 set.system.user	22

1.3.4 add.system.user	22

1.3.5 del.system.user	22

2. operator.cgi	23

2.1 VIDEO GENERAL	25

2.1.1 get.video.general	25

2.1.2 set.video.general	27

2.1.3 get.video.resolutions	27

2.1.4 get.video.bitrates	29

2.2 VIDEO ADVANCED	32

2.2.1 get.video.advanced	32

2.2.2 set.video.advanced	35

2.3 CAMERA SETTINGS	36

2.3.1 get.camera.setting	36

2.3.2 set.camera.setting	39

2.3.3 set.camera.loaddefault	39

2.3.4 set.camera.settings.bwmode (not implemented)	39

2.3.5 set.camera.settings.maxgain  (not implemented)	39

2.4 CAMERA PTZ	40

2.4.1 get.camera.ptz	40

2.4.2 set.camera.ptz	41

2.5 STORAGE DISK INFORMATION	42

2.5.1 get.recordfile.format (not implemented)	42

2.5.2 set.recordfile.format (not implemented)	43

2.5.3 set.storage.format (not implemented)	43

2.5.4 record.start (not implemented)	43

2.5.5 record.stop (not implemented)	43

2.5.6 record.continue (not implemented)	43

2.6 EVENT CONFIGURATION	44

2.6.1 get.event.configuration	44

2.6.2 update.event.configuration	49

2.6.3 add.event.configuration 	50

2.6.4 del.event.configuration	50

2.7 EVENT MOTION DETECTION	51

2.7.1 get.event.motion.detection	51

2.7.2 set.motion.osd (not implemented)	55

2.7.3 update.event.motion.detection	55

2.7.4 add.event.motion.detection	55

2.7.5 del.event.motion.detection	55

2.8 EVENT SERVER	56

2.8.1 get.event.server	56

2.8.2 update.event.ftp.server	58

2.8.3 add.event.ftp.server	59

2.8.4 del.event.ftp.server	59

2.8.5 update.event.http.server	59

2.8.6 add.event.http.server	59

2.8.7 del.event.http.server	59

2.8.8 update.event.tcp.server (not implemented)	59

2.8.9 add.event.tcp.server (not implemented)	60

2.8.10 del.event.tcp.server (not implemented)	60

2.8.11 update.event.smtp.server	60

2.8.12 add.event.smtp.server	60

2.8.13 del.event.smtp.server	60

2.8.14 check.smtp.email	61

2.9 EVENT SERVER I/O	62

2.9.1 get.event.io	62

2.9.3 active.event.io (not implemented)	64

2.10 EVENT PIR (not implemented)	65

2.10.1 get.event.pir (not implemented)	65

2.10.2 set.event.pir (not implemented)	68

2.11 YOICS (not implemented)	69

2.11.1 get.yoics.stat (not implemented)	69

2.11.2 set.yoics.stat (not implemented)	69

3. viewer.cgi	70

3.1 Live view	70

3.1.1 get.liveview	70

3.1.2 rotate.dptz (not implemented)	71

3.2 PTZ control	71

3.2.1 ptz.control	71

3.2.2 ptz.speed	73

3.3 System	73

3.3.1 get.setup	73

3.3.2 get.Model	73

4. Operator misc	75

4.1 filelist.cgi	75

4.1.1 file.list	75

4.1.2 del.files	78

4.1.3 playback	78

4.1.4 recycle.cgi	78

4.1.5 Snapshot	79

5. Misc CGI	80

5.1 notify.fcgi	80

5.1.1 set.notify	80

5.2 audio.fcgi (not implemented)	81

- Revision history

| Revision | Date | Description |
| --- | --- | --- |
| Draft | 2011/06/23 | Draft version |
| 1.0 | 2013/05/24 | Update version |
| 1.1 | 2014/06/27 | Update version |
| 1.2 | 2015/04/08 | Update version |
| 1.3 | 2015/11/04 | Update version |
| 1.4 | 2015/12/18 | Update version |

- Overview

This document specifies the parameters and configuration files for Milesight IP camera.

- Permission level

The permission levels column in each parameter table shows the required permission level to operate parameters.

To be able to perform an action on a parameter the user needs to have a permission level equal to or higher than the corresponding permission level of the parameter.

| Security level | Description |
| --- | --- |
| Anonymous | Access the HTTP service without authentication. |
| Viewer | Normal access right. |
| Operator | High access right. |
| Admin | Top access right. |

- Valid values

| Data Type | Valid values |
| --- | --- |
| Boolean | 0 / 1 |
| String | Any string |
| Domain name | A domain name format. Max 80 characters. |
| Host name | A hostnma |
| IP address | A string on the format xxx.xxx.xxx.xxx, where xxx is a number between 0 to 255. Example : 192.168.0.200 |
| MAC address | A string on the format xx: xx: xx: xx: xx: xx, where xx is a hexadecimal value. Example : 00:24:1D:20:8E:F1 |
| Network port | 1 … 65535 |
| An integer | Any number between -2147483647 (-2^31-1) and 2147483647 (2^31-1). |
| An unsigned integer | Any number between 0 and 4294967295 (2^32-1). |

- 1. admin.cgi

The administrator operation interface.

Permissions: Admin

Syntax:

http://<URL>/cgi-bin/admin/admin.cgi?action=< Action Name >[&<parameter=value>]

Action List:

| Action Type | Action Name | INF | RS | Description |
| --- | --- | --- | --- | --- |
| NETWORK | get.network.general | V |  | Get network general parameters. |
|  | set.network.general |  | V | Set network general parameters. |
|  | get.network.advanced | V |  | Get network advanced parameters. |
|  | set.network.advanced |  | V | Set network advanced parameters. |
|  | get.network.ddns | V |  | Get network DDNS parameters. |
|  | set.network.ddns |  |  | Set network DDNS parameters. |
|  | get.network.ipchange | V |  | Get network ipchange parameters. |
|  | set.network.ipchange |  |  | Set network ipchange parameters. |
|  | get.network.wireless | V |  | Get network wireless parameters. |
|  | set.network.wireless |  | V | Set network wireless parameters. |
|  | search.network.wireless |  |  | Search wireless access points. |
| SYSTEM | get.system.information | V |  | Get system information parameters. |
|  | set.system.information |  |  | Set system information parameters. |
|  | get.system.datetime | V |  | Get system date time parameters. |
|  | set.system.datetime |  |  | Se system date time parameters. |
|  | get.system.maintenance | V |  | Get system maintenance parameters. |
|  | reboot.system.maintenance |  | V | Reboot the system. |
|  | loaddefault.system.maintenance |  | V | Load default setting. |
|  | backupfile.system.maintenance |  |  | Back system settings. |
| USER | get.system.user | V |  | Get system user parameters. |
|  | set.system.user.anonymous |  |  | Set system to anonymous. |
|  | set.system.user |  | V | Set a user parameter. |
|  | add.system.user |  |  | Add a user. |
|  | del.system.user |  |  | Delete a user. |

Note :

INF : INF format output is supported.

- RS : System will be restarted after setting.

- 1.1 NETWORK

    - get.network.general

To get the general network parameters, such as basic Ethernet adapter, dns and PPPOE settings.

| Parameter | Values | Description |
| --- | --- | --- |
| model_hardware_wireless | Boolean | Wireless supported or not 0 : No 1 : Yes |
| network_interface_eth0_ipv4_dhcp | Boolean | Enable DHCP 0 : No 1 : Yes |
| network_interface_eth0_ipv4_manual_address | IP address | IP address for wired ethernet |
| network_interface_eth0_ipv4_manual_netmask | IP address | Net mask for wired ethernet |
| network_interface_eth0_ipv4_manual_gateway | IP address | Gateway for wired ethernet |
| network_dnsinformation_dnsmanual_0 | IP address | The 1st DNS |
| network_dnsinformation_dnsmanual_1 | IP address | The 2nd DNS |
| network_pppoe_enabled | Boolean | Enable PPPOE (not implemented) 0 : No 1 : Yes |
| network_pppoe_username | String, max 32 | PPPOE username |
| network_pppoe_password | String, max 32 | PPPOE password |
| network_vlan_enable | Boolean | Enable Vlan 0 : No 1 : Yes |
| network_vlan_id | An unsigned integer 1~ 4094 | Vlan id |
| network_vlan_ip | String, max 64 | Vlan ip |
| network_vlan_netmask | String, max 64 | Vlan netmask |
| network_vlan_gateway | String, max 64 | Vlan gateway |
| network_upnp_enable | Boolean | Enable upnp 0 : No 1 : Yes |
| network_upnp_name | String, max 63 | upnp name |
| network_upnp_type | Boolean | Type upnp 0 : auto 1 : manual |
| network_upnp_httpport | An unsigned integer 1 ~65535 | Upnp httpport |
| network_upnp_rtspport | An unsigned integer 1 ~65535 | Upnp rtspport |
| network_multicast_group_address | String, max 64 | Multicast group address |
| network_qos_dscp | An unsigned integer 0 ~ 63 | Qos Dscp |

Example :

http://192.168.0.200/cgi-bin/admin/admin.cgi?action=get.network.general

Return:

HTTP/1.1 200 OK

Cache-Control: no-store, no-cache, must-revalidate, post-check=0, pre-check=0, max-age=-1

Content-Type: text/html; charset=UTF-8

Transfer-Encoding: chunked

Date: Mon, 03 Jan 2000 04:32:18 GMT

Server: Boa/0.94.13

var model_hardware_wireless = '0'; var network_interface_ipv4_manual_address = '192.168.5.227'; var network_interface_ipv4_manual_netmask = '255.255.255.0'; var network_interface_ipv4_dhcp = '0'; var network_interface_ipv4_manual_gateway = '192.168.5.1'; var network_dnsinformation_dnsmanual_0 = '192.168.5.1'; var network_dnsinformation_dnsmanual_1 = ''; var ipchange_enabled = '0''; var network_pppoe_enabled = '0'; var network_pppoe_username = ''; var network_pppoe_password = '';

    - set.network.general

To set general network parameters.

Example: Set streams parameter

http://192.168.0.200/cgi-bin/admin/admin.cgi?action=set.network.general&network_interface_ipv4_dhcp=0&network_pppoe_enabled=0&network_pppoe_username=&network_pppoe_password=&network_interface_ipv4_manual_address=192.168.5.227&network_interface_ipv4_manual_netmask=255.255.255.0&network_interface_ipv4_manual_gateway=192.168.5.1&network_interface_ipv4_manual_prefixlength=24&network_dnsinformation_dnsmanual_0=192.168.5.1&network_dnsinformation_dnsmanual_1=192.168.5.1&network_vlan_id=1&network_vlan_ip=192.168.1.1&network_vlan_netmask=255.255.255.0&network_vlan_gateway=192.168.9.1&network_upnp_enable=0&network_upnp_name=David&network_upnp_type=0&network_upnp_httpport=21202&network_upnp_rtspport=23202&network_multicast_group_address=239.6.6.6&network_qos_dscp=10

    - get.network.advanced

To get advanced network settings parameters, such as NTP, hostname and network protocol ports.

| Parameter | Values | Description |
| --- | --- | --- |
| network_ntpinformation_fromdhcp (not implemented) | Boolean | NTP from DHCP 0 : No 1 : Yes |
| network_ntpinformation_ntpmanual | String, max 80 | NTP server address. |
| network_hostnameinformation_fromdhcp (not implemented) | Boolean | Hostname from DHCP 0 : No 1 : Yes |
| network_hostnameinformation_name | String, max 80 |  |
| network_protocol_http_enabled | Boolean | Enable HTTP. 0 : No, 1 : Yes |
| network_protocol_http_port | Network Port | HTTP port number. |
| network_protocol_https_enabled | Boolean | Enable HTTPS. 0 : No, 1 : Yes |
| network_protocol_https_port | Network Port | HTTPS port number. |
| network_protocol_arp_enabled (not implemented) | Boolean | Enable ARP. 0 : No, 1 : Yes |
| network_protocol_ftp_enabled (not implemented) | Boolean | Enable FTP. 0 : No, 1 : Yes |
| network_protocol_ftp_port(not implemented) | Network Port | FTP port number. |
| network_protocol_rtsp_enabled | 1 | Read only. RTSP server is always enabled. |
| network_protocol_rtsp_port | Network Port | RTSP port number. |
| network_protocol_upnp_enabled | Boolean | Enable UPNP. |
| network_protocol_bonjour_enabled | Boolean | Enable Bonjour. (not implemented) |
| network_protocol_rtspOverHttp_port | Network Port | RTSP over HTTP port number. |

Example :

http://192.168.0.200/cgi-bin/admin/admin.cgi?action=get.network.advanced

Return:

HTTP/1.1 200 OK

Cache-Control: no-store, no-cache, must-revalidate, post-check=0, pre-check=0, max-age=-1

Content-Type: text/html; charset=UTF-8

Transfer-Encoding: chunked

Date: Sat, 01 Jan 2000 00:32:59 GMT

Server: Boa/0.94.13

var model_hardware_wireless = '0'; var network_ntpinformation_fromdhcp = '0'; var network_ntpinformation_ntpmanual = 'pool.ntp.org'; var network_hostnameinformation_host_fromdhcp = '0'; var network_hostnameinformation_host_name = '192.168.4.238'; network_protocol_enabled[0] = '1'; network_protocol_name[0] = 'http'; network_protocol_port[0] = '8080'; network_protocol_enabled[1] = '0'; network_protocol_name[1] = 'https'; network_protocol_port[1] = ''; network_protocol_enabled[2] = '0'; network_protocol_name[2] = 'arp'; network_protocol_port[2] = ''; network_protocol_enabled[3] = '0'; network_protocol_name[3] = 'ftp'; network_protocol_port[3] = ''; network_protocol_enabled[4] = '1'; network_protocol_name[4] = 'rtsp'; network_protocol_port[4] = '554'; network_protocol_enabled[5] = '1'; network_protocol_name[5] = 'upnp'; network_protocol_port[5] = '1900'; network_protocol_enabled[6] = '0'; network_protocol_name[6] = 'bonjour'; network_protocol_port[6] = ''; network_protocol_enabled[7] = '0'; network_protocol_name[7] = 'rtspOverHttp'; network_protocol_port[7] = '8080'; var system_misc_haveyoics = '0';

- 1.1.4 set.network.advanced

To set advanced network parameters.

Example:

/cgi-bin/admin/admin.cgi?action=set.network.advanced&network_hostnameinformation_name=bruce.test&network_ntpinformation_ntpmanual=pool.ntp.org&network_protocol_http_port=8080&network_protocol_rtsp_port=554

- 1.1.5 get.network.ddns

To get Dynamic DNS parameters.

| Parameter | Values | Description |
| --- | --- | --- |
| network_ddns_enabled | Boolean | Enable DDNS. 0 : No 1 : Yes |
| network_ddns_hostname | String, max 80 | DDNS hostname |
| network_ddns_username | String, max 32 | DDNS username |
| network_ddns_password | String, max 32 | DDNS password |
| network_ddns_provider | String, max 32 | dyndns.org,freedns.afraid.org, www.no-ip.com,www.zoneedit.com |
| network_ddns_hash | String, max 32 | DDNS hash |

Example:

http://192.168.0.200/cgi-bin/admin/admin.cgi?action=get.network.ddns

Return:

HTTP/1.1 200 OK

Cache-Control: no-store, no-cache, must-revalidate, post-check=0, pre-check=0, max-age=-1

Content-Type: text/html; charset=UTF-8

Transfer-Encoding: chunked

Date: Mon, 03 Jan 2000 21:59:24 GMT

Server: Boa/0.94.13

var model_hardware_wireless = '0'; var network_ddns_enabled = '0'; var network_ddns_hostname = ''; var network_ddns_username = ''; var network_ddns_password = ''; var network_ddns_provider = ''; var network_ddns_hash = '';

- 1.1.6 set.network.ddns

To set Dynamic DNS parameters.

Example :

http://192.168.0.200/cgi-bin/admin/admin.cgi?action=set.network.ddns&network_ddns_enabled=1&network_ddns_hostname=your.host.name&network_ddns_username=username&network_ddns_password=password&network_ddns_provider=dyndns.org&network_ddns_hash=test

1.1.7 get.network.ipchange  (not implemented)

To get IP-changed notification parameters.

| parameter | Values | Description |
| --- | --- | --- |
| ipchange_enabled | Boolean | Enable IP-change Notification. 0 : No 1 : Yes |
| event_rule_actionemail_smtp_id | An unsigned integer | SMTP server ID. It’s related Event-Server configuration. |
| event_rule_actionemail_smtp_email | String, max 80 | The notification target email address. |

Example :

http://192.168.0.200/cgi-bin/admin/admin.cgi?action=get.network.ipchange

Return:

HTTP/1.1 200 OK

Cache-Control: no-store, no-cache, must-revalidate, post-check=0, pre-check=0, max-age=-1

Content-Type: text/html; charset=UTF-8

Transfer-Encoding: chunked

Date: Mon, 03 Jan 2000 22:04:59 GMT

Server: Boa/0.94.13

var model_hardware_wireless = '0'; var ipchange_enabled = '0'; var event_rule_actionemail_smtp_id = '0'; var event_rule_actionemail_smtp_email = '';

1.1.8 set.network.ipchange (not implemented)

To set IP-changed notification parameters.

Example :

http://192.168.0.200/cgi-bin/admin/admin.cgi?action=set.network.ipchange&ipchange_enabled=1&event_rule_actionemail_smtp_id=0&event_rule_actionemail_smtp_email=test%40gmail.com

- 1.1.9 get.network.wireless

To get Wireless setting parameters and Wireless-AP information.

| Parameter | Values | Description |
| --- | --- | --- |
| wireless_setting_enabled | Boolean | Enable Wireless 0 : No 1 : Yes |
| wireless_setting_op_mode | Boolean | Wireless network type (not implemented) 0 infrastructure, 1 Ad-Hoc |
| wireless_setting_ssid | String, max 32 | AP SSID ( url encode ) |
| wireless_setting_security | An unsigned integer 0 ~ 4 | Security mode 0 : None 1 : WEP 2 : WPA 3 : WPA2 4 : WPS-PIN |
| wireless_setting_web_authentication | Boolean | WEP authentication 0 Open,1 Shared key |
| wireless_setting_web_key_length | Boolean | WEP key length 0 64bits, 1 128bits |
| wireless_setting_web_key_mode | Boolean | WEP key type 0 Hex, 1 ASCII |
| wireless_setting_web_key_id | An unsigned integer 0 ~ 4 | WEP key ID |
| wireless_setting_web_key0_64m | String, max 10 | WEP 64bits Hex key |
| wireless_setting_web_key1_64m | String, max 10 |  |
| wireless_setting_web_key2_64m | String, max 10 |  |
| wireless_setting_web_key3_64m | String, max 10 |  |
| wireless_setting_web_key0_64a | String, max 5 | WEP 64bits ASCII key |
| wireless_setting_web_key1_64a | String, max 5 |  |
| wireless_setting_web_key2_64a | String, max 5 |  |
| wireless_setting_web_key3_64a | String, max 5 |  |
| wireless_setting_web_key0_128m | String, max 26 | WEP 128bits Hex key |
| wireless_setting_web_key1_128m | String, max 26 |  |
| wireless_setting_web_key2_128m | String, max 26 |  |
| wireless_setting_web_key3_128m | String, max 26 |  |
| wireless_setting_web_key0_128a | String, max 13 | WEP 128bits ASCII key |
| wireless_setting_web_key1_128a | String, max 13 |  |
| wireless_setting_web_key2_128a | String, max 13 |  |
| wireless_setting_web_key3_128a | String, max 13 |  |
| wireless_setting_wpa_encryption | Boolean | WPA encryption mode 0 : TKIP 1 : AES |
| wireless_setting_wpa_presharekey | String, max 63 | WPA key |
| wireless_setting_wpa2_encryption | Boolean | WPA2 encryption mode 0 : TKIP 1 : AES |
| wireless_setting_wpa2_presharekey | String, max 63 | WPA2 key |
| wireless_setting_wps_pin | String, max 128 | WPS pin code |

Example :

http://192.168.0.200/cgi-bin/admin/admin.cgi?action=get.network.wireless

Return: Without WIFI AP information.

HTTP/1.1 200 OK

Cache-Control: no-store, no-cache, must-revalidate, post-check=0, pre-check=0, max-age=-1

Content-Type: text/html; charset=UTF-8

Transfer-Encoding: chunked

Date: Thu, 16 Jun 2011 10:21:07 GMT

Server: Boa/0.94.13

var ipchange_enabled = '0';var wireless_setting_enabled = '0'; var wireless_setting_op_mode = '0'; var wireless_setting_ssid = 'SSID'; var wireless_setting_security = '0'; var wireless_setting_wep_authentication = '0'; var wireless_setting_wep_key_length = '1'; var wireless_setting_wep_key_mode = '0'; var wireless_setting_wep_key_id = '0'; var wireless_setting_wps_pin = '28664246'; var wireless_setting_wep_key0_64m = ''; var wireless_setting_wep_key1_64m = ''; var wireless_setting_wep_key2_64m = ''; var wireless_setting_wep_key3_64m = '';var wireless_setting_wep_key0_64a = ''; var wireless_setting_wep_key1_64a = ''; var wireless_setting_wep_key2_64a = ''; var wireless_setting_wep_key3_64a = '';var wireless_setting_wep_key0_128m = ''; var wireless_setting_wep_key1_128m = ''; var wireless_setting_wep_key2_128m = ''; var wireless_setting_wep_key3_128m = ''; var wireless_setting_wep_key0_128a = ''; var wireless_setting_wep_key1_128a = ''; var wireless_setting_wep_key2_128a = ''; var wireless_setting_wep_key3_128a = '';var wireless_setting_wpa_encryption = '0'; var wireless_setting_wpa_presharekey = ''; var wireless_setting_wpa2_encryption= '0'; var wireless_setting_wpa2_presharekey = '';var system_deviceinformation_NetInf = '17';

Return: With WIFI AP information

HTTP/1.1 200 OK

Cache-Control: no-store, no-cache, must-revalidate, post-check=0, pre-check=0, max-age=-1

Content-Type: text/html; charset=UTF-8

Transfer-Encoding: chunked

Date: Thu, 16 Jun 2011 10:21:38 GMT

Server: Boa/0.94.13

wireless_accesspoint_ssid[0] = 'AP001'; wireless_accesspoint_mode[0] = 'infrastructure'; wireless_accesspoint_enc[0] = 'TKIP'; wireless_accesspoint_auth[0] = 'WPA-PSK'; wireless_accesspoint_signals[0] = '63'; Wpsreless_accesspoint_wps[0] = 'NO';wireless_accesspoint_ssid[1] = 'ncqa'; wireless_accesspoint_mode[1] = 'infrastructure'; wireless_accesspoint_enc[1] = 'NONE'; wireless_accesspoint_auth[1] = 'OPEN'; wireless_accesspoint_signals[1] = '83'; Wpsreless_accesspoint_wps[1] = 'NO';wireless_accesspoint_ssid[2] = 'KSIP4'; wireless_accesspoint_mode[2] = 'infrastructure'; wireless_accesspoint_enc[2] = 'AES'; wireless_accesspoint_auth[2] = 'WPA2-PSK'; wireless_accesspoint_signals[2] = '100'; Wpsreless_accesspoint_wps[2] = 'NO';wireless_accesspoint_ssid[3] = ''AP002''; wireless_accesspoint_mode[3] = 'infrastructure'; wireless_accesspoint_enc[3] = 'TKIP'; wireless_accesspoint_auth[3] = 'WPA-PSK'; wireless_accesspoint_signals[3] = '83'; Wpsreless_accesspoint_wps[3] = 'YES';wireless_accesspoint_ssid[4] = 'Guest'; wireless_accesspoint_mode[4] = 'infrastructure'; wireless_accesspoint_enc[4] = 'NONE'; wireless_accesspoint_auth[4] = 'OPEN'; wireless_accesspoint_signals[4] = '78'; Wpsreless_accesspoint_wps[4] = 'NO';wireless_accesspoint_ssid[5] = 'nc%5Flinksys'; wireless_accesspoint_mode[5] = 'infrastructure'; wireless_accesspoint_enc[5] = 'AES'; wireless_accesspoint_auth[5] = 'WPA2-PSK'; wireless_accesspoint_signals[5] = '68'; Wpsreless_accesspoint_wps[5] = 'NO';wireless_accesspoint_ssid[6] = '%27XXX%27%2D1'; wireless_accesspoint_mode[6] = 'infrastructure'; wireless_accesspoint_enc[6] = 'TKIP + AES'; wireless_accesspoint_auth[6] = 'WPA(2)-PSK'; wireless_accesspoint_signals[6] = '100'; Wpsreless_accesspoint_wps[6] = 'YES';wireless_accesspoint_ssid[7] = 'default'; wireless_accesspoint_mode[7] = 'infrastructure'; wireless_accesspoint_enc[7] = 'NONE'; wireless_accesspoint_auth[7] = 'OPEN'; wireless_accesspoint_signals[7] = '47'; Wpsreless_accesspoint_wps[7] = 'NO';var ipchange_enabled = '0';var wireless_setting_enabled = '0'; var wireless_setting_op_mode = '0'; var wireless_setting_ssid = 'SSID'; var wireless_setting_security = '0'; var wireless_setting_wep_authentication = '0'; var wireless_setting_wep_key_length = '1'; var wireless_setting_wep_key_mode = '0'; var wireless_setting_wep_key_id = '0'; var wireless_setting_wps_pin = '28664246'; var wireless_setting_wep_key0_64m = ''; var wireless_setting_wep_key1_64m = ''; var wireless_setting_wep_key2_64m = ''; var wireless_setting_wep_key3_64m = '';var wireless_setting_wep_key0_64a = ''; var wireless_setting_wep_key1_64a = ''; var wireless_setting_wep_key2_64a = ''; var wireless_setting_wep_key3_64a = '';var wireless_setting_wep_key0_128m = ''; var wireless_setting_wep_key1_128m = ''; var wireless_setting_wep_key2_128m = ''; var wireless_setting_wep_key3_128m = ''; var wireless_setting_wep_key0_128a = ''; var wireless_setting_wep_key1_128a = ''; var wireless_setting_wep_key2_128a = ''; var wireless_setting_wep_key3_128a = '';var wireless_setting_wpa_encryption = '0'; var wireless_setting_wpa_presharekey = ''; var wireless_setting_wpa2_encryption= '0'; var wireless_setting_wpa2_presharekey = '';var system_deviceinformation_NetInf = '17';

- 1.1.10 set.network.wireless

To set Wireless parameters.

Example :

http://192.168.0.200/cgi-bin/admin/admin.cgi?action=set.network.wireless&wireless_setting_enabled=0&wireless_setting_op_mode=0&wireless_setting_ssid=nc_linksys&wireless_setting_security=3&wireless_setting_web_authentication=0&wireless_setting_web_key_length=1&wireless_setting_web_key_mode=0&wireless_setting_web_key_id=0&wireless_setting_web_key0_64m=&wireless_setting_web_key1_64m=&wireless_setting_web_key2_64m=&wireless_setting_web_key3_64m=&wireless_setting_web_key0_64a=&wireless_setting_web_key1_64a=&wireless_setting_web_key2_64a=&wireless_setting_web_key3_64a=&wireless_setting_web_key0_128m=&wireless_setting_web_key1_128m=&wireless_setting_web_key2_128m=&wireless_setting_web_key3_128m=&wireless_setting_web_key0_128a=&wireless_setting_web_key1_128a=&wireless_setting_web_key2_128a=&wireless_setting_web_key3_128a=&wireless_setting_wpa_encryption=0&wireless_setting_wpa_presharekey=&wireless_setting_wpa2_encryption=1&wireless_setting_wpa2_presharekey=111&wireless_setting_wps_pin=28664246

- 1.1.11 search.network.wireless

To search or refresh Wireless AP information.

Example :

http://192.168.0.200/cgi-bin/admin/admin.cgi?action=search.network.wireless

- 1.2 SYSTEM

- 1.2.1 get.system.information

To get system information.

| Parameter | Values | Description |
| --- | --- | --- |
| system_misc_devicename | String, max 60 | Device name |
| system_misc_devicelocation | String, max 60 | Device location (not implemented) |
| system_hardware_version | String | Hardware version |
| system_software_version | String | Software version |
| system_kernel_version | String | Kernel version |
| system_io_support | Boolean | The device support alarm input and output moudle 1:support 0:unsupport |
| system_audio_support | Boolean | The device support alarm input and output moudle 1:support 0:unsupport |

Example:

http://192.168.0.200/cgi-bin/admin/admin.cgi?action=get.system.information

Return:

HTTP/1.1 200 OK

Cache-Control: no-store, no-cache, must-revalidate, post-check=0, pre-check=0, max-age=-1

Content-Type: text/html; charset=UTF-8

Transfer-Encoding: chunked

Date: Thu, 16 Jun 2011 11:35:07 GMT

Server: Boa/0.94.13

var MODEL = 'MS-C3351'; var system_deviceinformation_firmwareversion = '22.4.3.27'; var system_deviceinformation_systemboottime = '05/24/2013 10:11:41'; var system_deviceinformation_wireless = '0'; network_interface_info_hwaddress[0] = '1c:c3:16:02:00:2a'; network_interface_ipv4_dhcp[0] = '0'; network_interface_ipv4_current_address[0] = '192.168.5.227'; network_interface_ipv4_current_netmask[0] = '255.255.255.0'; network_interface_ipv4_current_gateway[0] = '192.168.5.1'; network_pppoe_pppoe_enabled  = '0'; var network_dnsinformation_dnsmanual_0 = '192.168.5.1'; var network_dnsinformation_dnsmanual_1 = ''; var network_ddns_enabled = '0'; var network_ddns_hostname = ''; var network_ddns_status = '0'; var wireless_setting_enabled = '0'; var system_misc_devicename = 'DeviceName'; var system_misc_devicelocation = 'XiaMen China'; var system_deviceinformation_vender = 'Milesight'; var system_hardware_version = ‘V1.4’; var system_software_version = ‘20.5.0.101’; var system_kernel_version = ‘2.2.0.7’;

- 1.2.2 set.system.information

To set system device name and location.

Example:

http://192.168.0.200/cgi-bin/admin/admin.cgi?action=set.system.information&system_misc_devicename=DeviceName&system_misc_devicelocation=DeviceLocation

- 1.2.3 get.system.datetime

To get system date and time.

| Parameter | Values | Description |
| --- | --- | --- |
| system_datetime_datetimetype | An unsigned integer 0~2 | Set date/time mode 0 : Manual ( sync with PC ) 1 : NTP 2 : Manual |
| system_datetime_timezone_tz | String, max 8 -12:00 -11:00 …. 0:00 …. +12:00 +13:00 | Time zone |
| system_datetime_localdatetime_time_hour | An unsigned integer 0~23 | Time hour |
| system_datetime_localdatetime_time_minute | An unsigned integer 0~59 | Time minute |
| system_datetime_localdatetime_time_second | An unsigned integer 0~59 | Time second |
| system_datetime_localdatetime_date_year | Integer | Date year |
| system_datetime_localdatetime_date_month | An unsigned integer 1~12 | Date month |
| system_datetime_localdatetime_date_day | An unsigned integer 1~31 | Date day |
| system_datetime_ntp_sync_enable | Boolean | ntp sync enable 0: Disable 1: enable |
| system_datetime_ntp_interval | An unsigned integer 0~8 | ntp interval 0:1 hour 1:2 hours 2:5 hours 3:10 hours 4:1 day 5:2 days 6:5 days 7:10 days 8:30 days |

Example :

http://192.168.0.200/cgi-bin/admin/admin.cgi?action=get.system.datetime

Return:

HTTP/1.1 200 OK

Cache-Control: no-store, no-cache, must-revalidate, post-check=0, pre-check=0, max-age=-1

Content-Type: text/html; charset=UTF-8

Transfer-Encoding: chunked

Date: Thu, 16 Jun 2011 11:41:03 GMT

Server: Boa/0.94.13

var system_datetime_localdatetime_date_year = '2013'; var system_datetime_localdatetime_date_month = '5'; var var system_datetime_localdatetime_date_day = '24'; var system_datetime_localdatetime_time_hour = '13'; var system_datetime_localdatetime_time_minute = '6'; var system_datetime_localdatetime_time_second = '35'; var system_datetime_timezone_tz  = '-8:00'; var system_datetime_datetimetype = '1'; var system_datetime_id = '0'

- 1.2.4 set.system.datetime

To set system date and time.

Example:

http://192.168.0.200/cgi-bin/admin/admin.cgi?action=set.system.datetime&system_datetime_datetimetype=0&system_datetime_timezone_tz=+8:00&system_datetime_localdatetime_time_hour=11&system_datetime_localdatetime_time_minute=41&system_datetime_localdatetime_time_second=8&system_datetime_localdatetime_date_year=2011&system_datetime_localdatetime_date_month=6&system_datetime_localdatetime_date_day=16&system_datetime_id=29&system_datetime_ntp_sync_enable=1&system_datetime_ntp_interval=4

- 1.2.5 get.system.maintenance

To get system maintenance information.

http://192.168.0.200/cgi-bin/admin/admin.cgi?action=get.system.maintenance

| Parameter | Values | Description |
| --- | --- | --- |
| MODEL | String | Read only. Model name. |
| system_deviceinformation_firmwareversion | String | Read only. Firmware version. |
| network_interface_info_hwaddress | String | Read only. MAC address. |

Return:

HTTP/1.1 200 OK

Cache-Control: no-store, no-cache, must-revalidate, post-check=0, pre-check=0, max-age=-1

Content-Type: text/html; charset=UTF-8

Transfer-Encoding: chunked

Date: Thu, 16 Jun 2011 13:23:07 GMT

Server: Boa/0.94.13

var MODEL = 'MS-C3351'; var system_deviceinformation_firmwareversion = '22.4.3.27'; var network_interface_info_hwaddress = '1c:c3:16:02:00:2a';

- 1.2.6 reboot.system.maintenance

Request to reboot system.

http://192.168.0.200/cgi-bin/admin/admin.cgi?action=reboot.system.maintenance

- 1.2.7 loaddefault.system.maintenance

Request to load default settings and reboot the system.

http://192.168.0.200/cgi-bin/admin/admin.cgi?action=loaddefault.system.maintenance

- 1.2.8 backupfile.system.maintenance (not implemented)

Backup system setting, /backup.zip will be create when this action has been requested.

Step 1 : Request this action.

Step 2 : Down load backup file. ( http://192.168.0.200/backup.zip ) (not implemented)

http://192.168.0.200/cgi-bin/admin/admin.cgi?action=backupfile.system.maintenance

- 1.3 USER

- 1.3.1 get.system.user

To get system user list

| Parameter | Values | Description |
| --- | --- | --- |
| security_user_password | String, max 16 | User password |
| security_user_userlevel | An unsigned integer 0~2 | New user level 0 : Administrator 1 : Operator 2 : Viewer |
| security_user_username | String, max 32 | User name |

http://192.168.0.200/cgi-bin/admin/admin.cgi?action=get.system.user

Return: Only one user

HTTP/1.1 200 OK

Cache-Control: no-store, no-cache, must-revalidate, post-check=0, pre-check=0, max-age=-1

Content-Type: text/html; charset=UTF-8

Transfer-Encoding: chunked

Date: Thu, 16 Jun 2011 13:40:25 GMT

Server: Boa/0.94.13

var system_misc_anonymous = '0';security_user_username[0] = 'admin'; security_user_userlevel[0] = '0';

Return: Two user

HTTP/1.1 200 OK

Cache-Control: no-store, no-cache, must-revalidate, post-check=0, pre-check=0, max-age=-1

Content-Type: text/html; charset=UTF-8

Transfer-Encoding: chunked

Date: Sat, 01 Jan 2000 01:00:36 GMT

Server: Boa/0.94.13

var system_misc_anonymous = '0';security_user_username[0] = 'admin'; security_user_userlevel[0] = '0';security_user_username[1] = 'aaaa'; security_user_userlevel[1] = '0';

- 1.3.2 set.system.user.anonymous

To set anonymous for HTTP service.

| Parameter | Values | Description |
| --- | --- | --- |
| system_misc_anonymous | Boolean | Enable anonymous 0 : No 1 : Yes |

Example:

http://192.168.0.200/cgi-bin/admin/admin.cgi?action=set.system.user.anonymous&system_misc_anonymous=1

- 1.3.3 set.system.user

To update a user account parameter.

| Parameter | Values | Description |
| --- | --- | --- |
| security_user_password | String, max 16 | New user password |
| security_user_userlevel | An unsigned integer 0~2 | New user level 0 Admin, 1 Operator, 2 Viewer |
| security_user_username | String, max 32 | Username (already exist in the user list) |

Example:

http://192.168.0.200/cgi-bin/admin/admin.cgi?action=set.system.user&security_user_username=admin&security_user_password=password&security_user_userlevel=0

- 1.3.4 add.system.user

To create a new user account.

Example:

http://192.168.0.200/cgi-bin/admin/admin.cgi?action=add.system.user&security_user_username=test&security_user_password=test&security_user_userlevel=2

- 1.3.5 del.system.user

To delete a user account.

| Parameter | Values | Description |
| --- | --- | --- |
| security_user_username | String, max 32 | Username (already exist in the user list) |

Example:

http://192.168.0.200/cgi-bin/admin/admin.cgi?action=del.system.user&security_user_username=test

- 2. operator.cgi

Permissions: Admin, Operator

Usage:

http://<URL>/cgi-bin/operator/operator.cgi?action=value>[&<parameter=value>]

Action List:

| Action Type | Action Name | INF | RS | Description |
| --- | --- | --- | --- | --- |
| VIDEO GENERAL | get.video.general | V |  | Get streams parameter |
|  | set.video.general |  | V | Set streams parameter |
| VIDEO ADVANCED | get.video.advanced | V |  | Get video advanced parameters. Such as DPTZ / Audio / OSD |
|  | set.video.advanced |  | V | Set video advanced parameters. |
| CAMERA SETTINGS | get.camera.setting | V |  | Get camera parameters. AE / WB / Mirror … etc. |
|  | set.camera.setting |  |  | Set camera parameters. |
|  | set.camera.loaddefault |  | V | Load default setting. |
|  | set.camera.settings.bwmode |  |  | Set B/W mode. |
|  | set.camera.settings.maxgain |  |  | Set max gain. |
| CAMERA PTZ | get.camera.ptz | V |  | Get PTZ parameter. |
|  | set.camera.ptz |  |  | Set PTZ parameter. |
| STORAGE DISK INFORMATION | get.recordfile.format | V |  | Get redording parameter. |
|  | set.recordfile.format |  |  | Set redording parameter |
|  | set.storage.format |  |  | Format SD card |
|  | record.start |  |  | Start recording |
|  | record.stop |  |  | Stop recording |
|  | record.continue |  |  | Stop current record file and start new record file |
| EVENT CONFIGURATION | get.event.configuration | V |  | Get event configuration information. |
|  | update.event.configuration |  |  | Update an event rule. |
|  | add.event.configuration |  |  | Create an event rule. |
|  | del.event.configuration |  |  | Delete an event rule. |
| EVENT MOTION DETECTION | get.event.motion.detection | V |  | Get motion detection parameters. |
|  | set.motion.osd |  |  | Set motion OSD. |
|  | update.event.motion.detection |  |  | Update a motion window. |
|  | add.event.motion.detection |  |  | Create a motion window. |
|  | del.event.motion.detection |  |  | Delete a motion window. |
| EVENT SERVER | get.event.server | V |  | Get event server informationi. |
|  | update.event.ftp.server |  |  | Update FTP server parameters. |
|  | add.event.ftp.server |  |  | Create a new FTP server. |
|  | del.event.ftp.server |  |  | Delete a FTP server. |
|  | update.event.http.server |  |  | Update HTTP server parameters. |
|  | add.event.http.server |  |  | Create a new HTTP server. |
|  | del.event.http.server |  |  | Delete a HTTP server. |
|  | update.event.tcp.server |  |  | Update TCP server parameters. |
|  | add.event.tcp.server |  |  | Create a new TCP server. |
|  | del.event.tcp.server |  |  | Delete a TCP server. |
|  | update.event.smtp.server |  |  | Update SMTP server parameters. |
|  | add.event.smtp.server |  |  | Create a SMTP server. |
|  | del.event.smtp.server |  |  | Delete a SMTP server. |
|  | check.smtp.email |  |  | Check SMTP email function. |
| EVENT SERVER I/O | get.event.io | V |  | Get event I/O nformation. |
|  | set.event.io |  |  | Set event I/O function. |
|  | active.event.io |  |  | Active event I/O. |
| ..EVENT PIR | get.event.pir | V |  | Get PIR parameters. |
|  | set.event.pir |  |  | Set PIR parameters. |
| YOICS | get.yoics.stat | V |  | Get YOICS status. |
|  | set.yoics.stat |  |  | Set YOICS status. |
| PIR | testpir |  |  | Test PIR function. |

- 2.1 VIDEO GENERAL

- 2.1.1 get.video.general

To get streams parameter

| Parameter | Values | Description |
| --- | --- | --- |
| media_profile_resolution_width_<ch> | 1600x1200 1280x720 | (Only for 2M sensor series) (Only for channel 1 & 2) Ch1&2 changes at the same time. |
| media_profile_resolution_height_<ch> |  |  |
| media_profile_url_<ch> | String, max 10 | Stream path ( URI ) Fixed value |
| media_profile_framerate_<ch> | An unsigned integer 1~30 (25) | Stream frame rate |
| media_profile_gop_<ch> | An unsigned integer 1~60 | H264 GOP size |
| media_profile_ratecontrol_mode_<ch> | Boolean | H264 compression mode 0 : CBR 1 : VBR |
| media_profile_ratecontrol_targetbitrate_<ch> | An unsigned integer | H264 bitrate (for 5MP(2560x1920):16~6144kbps For 3MP(2560x1920):16~6144kbps for HD: 16~16384Kbps for VGA/NTSC/PAL: 16~1024 Kbps for QVGA: 16~1024 Kbps for QQVG: 16~1024 Kbps ) |
| media_profile_ratecontrol_quality_<ch> | An unsigned integer 0,1,2 | Stream quality level 0 : Best 1 : Good 2 : Standard |
| media_profile_profile_<ch> | Boolean | H264 profile level 0 : Baseline profile 1 : Main profile 2 : High profile |
| media_profile_codec_1 | An unsigned integer 0,1,2 | Stream 1 is H.264.（Fixed value） 0:H.264 1:MPEG4 2:MJPEG |
| media_profile_enabled_<ch> | Boolean | Stream 1 or 2 is enable 0:disable 1:enable |

注：次码流使能，ch有设置1，则认为开启次码流。

<ch>从0小标开始

Usage:

http://<serverIP>/cgi-bin/operator/operator.cgi?action=get.video.general[&<parameter>=<value>…]

| Parameter | Values | Description |
| --- | --- | --- |
| format | inf | Export INF format |

Example: Get streams parameter

http://192.168.0.200/cgi-bin/operator/operator.cgi?action=get.video.general&format=inf

Return: INF format

HTTP/1.1 200 OK

Cache-Control: no-store, no-cache, must-revalidate, post-check=0, pre-check=0, max-age=-1

Content-Type: text/plain; charset=UTF-8

Transfer-Encoding: chunked

Date: Sat, 01 Jan 2000 02:53:42 GMT

Server: Boa/0.94.13

[VIDEO_GENERAL]

system_deviceinformation_model=MS-C3351

system_deviceinformation_sensor=3m_0331

system_deviceinformation_tvstandards=-1

media_profile_resolution_width_0=1280

media_profile_resolution_height_0=720

media_profile_url_0=main

media_profile_gop_0=20

media_profile_ratecontrol_mode_0=1

media_profile_framerate_0=0

media_profile_ratecontrol_targetBitrate_0=2048

media_profile_ratecontrol_quality_0=0

media_profile_profile_0=1

media_profile_resolution_width_1=640

media_profile_resolution_height_1=480

media_profile_url_1=sub

media_profile_gop_1=30

media_profile_ratecontrol_mode_1=1

media_profile_framerate_1=1

media_profile_ratecontrol_targetBitrate_1=256

media_profile_ratecontrol_quality_1=75

media_profile_profile_1=0

cTVStandards=0

rtsp_port=555

Example: Get streams parameter

http://192.168.0.200/cgi-bin/operator/operator.cgi?action=get.video.general

Return: Normal format

HTTP/1.1 200 OK

Cache-Control: no-store, no-cache, must-revalidate, post-check=0, pre-check=0, max-age=-1

Content-Type: text/html; charset=UTF-8

Transfer-Encoding: chunked

Date: Wed, 15 Jun 2011 11:24:17 GMT

Server: Boa/0.94.13

var system_deviceinformation_model = 'MS-C3351'; var system_deviceinformation_sensor = '3m_0331'; var system_deviceinformation_tvstandards = '-1'; media_profile_resolution_width[0] = '1280'; media_profile_resolution_height[0] = '720'; media_profile_url[0] = 'main'; media_profile_gop[0] = '20'; media_profile_ratecontrol_mode[0] = '1'; media_profile_framerate[0] = '0'; media_profile_ratecontrol_targetBitrate[0] = '2048'; media_profile_ratecontrol_quality[0] = '0'; media_profile_profile[0] = '1'; media_profile_resolution_width[1] = '640'; media_profile_resolution_height[1] = '480'; media_profile_url[1] = 'sub'; media_profile_gop[1] = '30'; media_profile_ratecontrol_mode[1] = '1'; media_profile_framerate[1] = '1'; media_profile_ratecontrol_targetBitrate[1] = '256'; media_profile_ratecontrol_quality[1] = '75'; var media_profile_profile[1] = '0'; var cTVStandards = '0'; var rtsp_port = '555';

- 2.1.2 set.video.general

To set streams parameter.

with the following parameters and values:

“ch” range for non 2M sensor series: is 1 ~ 6

for other series is 1 ~ 8

ch 1, 3, 5, 7 are H264

ch 2, 4, 6, 8 are MJPEG

没有这些限制，且只有主次码流

Note : The system will restart if the resolution is changed.

Example: Set streams parameter

http://192.168.0.200/cgi-bin/operator/operator.cgi?action=set.video.general&media_profile_resolution_width_0=1280&media_profile_resolution_height_0=720&media_profile_framerate_0=30&media_profile_gop_0=15&media_profile_ratecontrol_mode_0=1&media_profile_ratecontrol_targetbitrate_0=2048&media_profile_resolution_width_1=640&media_profile_resolution_height_1=480&media_profile_framerate_1=25&media_profile_gop_1=30&media_profile_ratecontrol_mode_1=1&media_profile_ratecontrol_targetbitrate_1=256

- 2.1.3 get.video.resolutions

To get streams support resolutions

| Parameter | Values | Description |
| --- | --- | --- |
| media_resolution_width_supported_<ch>_<index> | 1600x1200 1280x720 | Ch from 0 to 2, 0 means main stream, 1 means sub stream, 2 means third stream(if supported) |
| media_resolution_height_supported_<ch>_<index> |  |  |

Usage:

http://<serverIP>/cgi-bin/operator/operator.cgi?action=get.video.resolutions[&<parameter>=<value>…]

| Parameter | Values | Description |
| --- | --- | --- |
| format | inf | Export INF format |

Example: Get streams resolutions

http://192.168.0.200/cgi-bin/operator/operator.cgi?action=get.video.resolutions&format=inf

Return: INF format

HTTP/1.1 200 OK

Cache-Control: no-store, no-cache, must-revalidate, post-check=0, pre-check=0, max-age=-1

Content-Type: text/plain; charset=UTF-8

Transfer-Encoding: chunked

Date: Sat, 01 Jan 2000 02:53:42 GMT

Server: Boa/0.94.13

[VIDEO_GENERAL]

media_resolution_width_supported_0[0]=2048

media_resolution_height_supported_0[0]=1536

media_resolution_width_supported_0[1]=1920

media_resolution_height_supported_0[1]=1080

media_resolution_width_supported_0[2]=1600

media_resolution_height_supported_0[2]=1200

media_resolution_width_supported_0[3]=1280

media_resolution_height_supported_0[3]=960

media_resolution_width_supported_0[4]=1280

media_resolution_height_supported_0[4]=720

media_resolution_width_supported_0[5]=704

media_resolution_width_supported_1[0]=704

media_resolution_height_supported_1[0]=576

media_resolution_width_supported_1[1]=640

media_resolution_height_supported_1[1]=480

media_resolution_width_supported_1[2]=640

media_resolution_height_supported_1[2]=360

media_resolution_width_supported_1[3]=352

media_resolution_height_supported_1[3]=288

media_resolution_width_supported_1[4]=320

media_resolution_height_supported_1[4]=240

media_resolution_width_supported_1[5]=320

media_resolution_height_supported_1[5]=192

media_resolution_width_supported_1[6]=320

media_resolution_height_supported_1[6]=176

media_resolution_width_supported_2[0]=1280

media_resolution_height_supported_2[0]=720

media_resolution_width_supported_2[1]=704

media_resolution_height_supported_2[1]=576

media_resolution_width_supported_2[2]=640

media_resolution_height_supported_2[2]=480

media_resolution_width_supported_2[3]=640

media_resolution_height_supported_2[3]=360

media_resolution_width_supported_2[4]=320

media_resolution_height_supported_2[4]=240

media_resolution_width_supported_2[5]=320

media_resolution_height_supported_2[5]=192

media_resolution_width_supported_2[6]=320

media_resolution_height_supported_2[6]=176

Example: Get streams resolutions

http://192.168.0.200/cgi-bin/operator/operator.cgi?action=get.video.resolutions

Return: Normal format

HTTP/1.1 200 OK

Cache-Control: no-store, no-cache, must-revalidate, post-check=0, pre-check=0, max-age=-1

Content-Type: text/html; charset=UTF-8

Transfer-Encoding: chunked

Date: Wed, 15 Jun 2011 11:24:17 GMT

Server: Boa/0.94.13

var media_resolution_width_supported_0_0 = 2048; var media_resolution_height_supported_0_0 = 1536; var media_resolution_width_supported_0_1 = 1920; var media_resolution_height_supported_0_1 = 1080; var media_resolution_width_supported_0_2 = 1600; var media_resolution_height_supported_0_2 = 1200; var media_resolution_width_supported_0_3 = 1280; var media_resolution_height_supported_0_3 = 960; var media_resolution_width_supported_0_4 = 1280; var media_resolution_height_supported_0_4 = 720; var media_resolution_width_supported_0_5 = 704; var media_resolution_height_supported_0_5 = 576; var media_resolution_width_supported_1_0 = 704; var media_resolution_height_supported_1_0 = 576; var media_resolution_width_supported_1_1 = 640; var media_resolution_height_supported_1_1 = 480; var media_resolution_width_supported_1_2 = 640; var media_resolution_height_supported_1_2 = 360; var media_resolution_width_supported_1_3 = 352; var media_resolution_height_supported_1_3 = 288; var media_resolution_width_supported_1_4 = 320; var media_resolution_height_supported_1_4 = 240; var media_resolution_width_supported_1_5 = 320; var media_resolution_height_supported_1_5 = 192; var media_resolution_width_supported_1_6 = 320; var media_resolution_height_supported_1_6 = 176; var media_resolution_width_supported_2_0 = 1280; var media_resolution_height_supported_2_0 = 720; var media_resolution_width_supported_2_1 = 704; var media_resolution_height_supported_2_1 = 576; var media_resolution_width_supported_2_2 = 640; var media_resolution_height_supported_2_2 = 480; var media_resolution_width_supported_2_3 = 640; var media_resolution_height_supported_2_3 = 360; var media_resolution_width_supported_2_4 = 320; var media_resolution_height_supported_2_4 = 240; var media_resolution_width_supported_2_5 = 320; var media_resolution_height_supported_2_5 = 192; var media_resolution_width_supported_2_6 = 320; var media_resolution_height_supported_2_6 = 176;

- 2.1.4 get.video.bitrates

To get streams support bitrates

| Parameter | Values | Description |
| --- | --- | --- |
| media_bitrate_supported_<ch>_<index> |  | Ch from 0 to 2, 0 means main stream, 1 means sub stream, 2 means third stream(if supported) |

Usage:

http://<serverIP>/cgi-bin/operator/operator.cgi?action=get.video.bitrates[&<parameter>=<value>…]

| Parameter | Values | Description |
| --- | --- | --- |
| format | inf | Export INF format |

Example: Get streams bitrates

http://192.168.0.200/cgi-bin/operator/operator.cgi?action=get.video.bitrates&format=inf

Return: INF format

HTTP/1.1 200 OK

Cache-Control: no-store, no-cache, must-revalidate, post-check=0, pre-check=0, max-age=-1

Content-Type: text/plain; charset=UTF-8

Transfer-Encoding: chunked

Date: Sat, 01 Jan 2000 02:53:42 GMT

Server: Boa/0.94.13

[VIDEO_BITRATES]

media_bitrate_supproted_0[0] = 16

media_bitrate_supproted_0[1] = 32

media_bitrate_supproted_0[2] = 64

media_bitrate_supproted_0[3] = 128

media_bitrate_supproted_0[4] = 256

media_bitrate_supproted_0[5] = 512

media_bitrate_supproted_0[6] = 640

media_bitrate_supproted_0[7] = 768

media_bitrate_supproted_0[8] = 1024

media_bitrate_supproted_0[9] = 1536

media_bitrate_supproted_0[10] = 2048

media_bitrate_supproted_0[11] = 3072

media_bitrate_supproted_0[12] = 4096

media_bitrate_supproted_0[13] = 5120

media_bitrate_supproted_0[14] = 6144

media_bitrate_supproted_0[15] = 7168

media_bitrate_supproted_0[16] = 8192

media_bitrate_supproted_0[17] = 10240

media_bitrate_supproted_0[18] = 12288

media_bitrate_supproted_0[19] = 14336

media_bitrate_supproted_0[20] = 16384

media_bitrate_supproted_1[0] = 16

media_bitrate_supproted_1[1] = 32

media_bitrate_supproted_1[2] = 64

media_bitrate_supproted_1[3] = 128

media_bitrate_supproted_1[4] = 256

media_bitrate_supproted_1[5] = 384

media_bitrate_supproted_1[6] = 512

media_bitrate_supproted_1[7] = 640

media_bitrate_supproted_1[8] = 768

media_bitrate_supproted_1[9] = 1024

media_bitrate_supproted_1[10] = 1536

media_bitrate_supproted_1[11] = 2048

media_bitrate_supproted_2[0] = 16

media_bitrate_supproted_2[1] = 32

media_bitrate_supproted_2[2] = 64

media_bitrate_supproted_2[3] = 128

media_bitrate_supproted_2[4] = 256

media_bitrate_supproted_2[5] = 384

media_bitrate_supproted_2[6] = 512

media_bitrate_supproted_2[7] = 640

media_bitrate_supproted_2[8] = 768

media_bitrate_supproted_2[9] = 1024

media_bitrate_supproted_2[10] = 1536

media_bitrate_supproted_2[11] = 2048

Example: Get streams bitrates

http://192.168.0.200/cgi-bin/operator/operator.cgi?action=get.video.bitrates

Return: Normal format

HTTP/1.1 200 OK

Cache-Control: no-store, no-cache, must-revalidate, post-check=0, pre-check=0, max-age=-1

Content-Type: text/html; charset=UTF-8

Transfer-Encoding: chunked

Date: Wed, 15 Jun 2011 11:24:17 GMT

Server: Boa/0.94.13

var media_bitrate_supproted_0_0 = 16; var media_bitrate_supproted_0_1 = 32; var media_bitrate_supproted_0_2 = 64; var media_bitrate_supproted_0_3 = 128; var media_bitrate_supproted_0_4 = 256; var media_bitrate_supproted_0_5 = 512; var media_bitrate_supproted_0_6 = 640; var media_bitrate_supproted_0_7 = 768; var media_bitrate_supproted_0_8 = 1024; var media_bitrate_supproted_0_9 = 1536; var media_bitrate_supproted_0_10 = 2048; var media_bitrate_supproted_0_11 = 3072; var media_bitrate_supproted_0_12 = 4096; var media_bitrate_supproted_0_13 = 5120; var media_bitrate_supproted_0_14 = 6144; var media_bitrate_supproted_0_15 = 7168; var media_bitrate_supproted_0_16 = 8192; var media_bitrate_supproted_0_17 = 10240; var media_bitrate_supproted_0_18 = 12288; var media_bitrate_supproted_0_19 = 14336; var media_bitrate_supproted_0_20 = 16384; var media_bitrate_supproted_1_0 = 16; var media_bitrate_supproted_1_1 = 32; var media_bitrate_supproted_1_2 = 64; var media_bitrate_supproted_1_3 = 128; var media_bitrate_supproted_1_4 = 256; var media_bitrate_supproted_1_5 = 384; var media_bitrate_supproted_1_6 = 512; var media_bitrate_supproted_1_7 = 640; var media_bitrate_supproted_1_8 = 768; var media_bitrate_supproted_1_9 = 1024; var media_bitrate_supproted_1_10 = 1536; var media_bitrate_supproted_1_11 = 2048; var media_bitrate_supproted_2_0 = 16; var media_bitrate_supproted_2_1 = 32; var media_bitrate_supproted_2_2 = 64; var media_bitrate_supproted_2_3 = 128; var media_bitrate_supproted_2_4 = 256; var media_bitrate_supproted_2_5 = 384; var media_bitrate_supproted_2_6 = 512; var media_bitrate_supproted_2_7 = 640; var media_bitrate_supproted_2_8 = 768; var media_bitrate_supproted_2_9 = 1024; var media_bitrate_supproted_2_10 = 1536; var media_bitrate_supproted_2_11 = 2048;

- 2.2 VIDEO ADVANCED

- 2.2.1 get.video.advanced

To get other stream parameter.

DPTZ(数字云台)是英文 Digital Pan Tilt Zoom的缩写，我们不支持数字云台

| Parameter | Values | Description |
| --- | --- | --- |
| imaging_settings_osd_enabled | Boolean | Enable OSD 0 : No 1 : Yes |
| imaging_settings_osd_text_enabled | Boolean | Enable text OSD 0 : No 1 : Yes |
| imaging_settings_osd_text_string | String, max 20 | Text OSD string |
| imaging_settings_osd_datetime_enabled | Boolean | Enable Date & Time OSD 0 : No 1 : Yes |
| media_profile_dptz_id (not implemented) | 2,4 | DPTZ video source ID 2: channel 3&4 4: channel 5&6 |
| media_profile_dptz_enabled (not implemented) | Boolean | Enable DPTZ 0 : No 1 : Yes |
| media_profile_dptz_horzoffset (not implemented) | An unsigned integer | DPTZ top left X coordinates |
| media_profile_dptz_vertoffset (not implemented) | An unsigned integer | DPTZ top left Y coordinates |
| media_profile_dptz_width (not implemented) | An unsigned integer | DPTZ width |
| media_profile_dptz_height (not implemented) | An unsigned integer | DPTZ height |
| system_misc_audiooutvolume | An integer -73 ~ 6 | Audio output volume (db) 6 is mean +6db |
| audio_setting_enabled |  | Enable audio-in source |
| audio_setting_codec | An unsigned integer 1 ~ 3 | Select audio encode format 1 : AAC 2 : G.711 u-Law 3 : G.711 a-Law |
| audio_setting_source | Boolean | Select audio-in source 0 : MIC 1 : Line-in |
| audio_setting_mode | An unsigned integer 0~ 2 | Audio mode 0: only mic 1: only speaker 2: both mic and speaker |
| audio_setting_denoise | Boolean | Audio denoise |
| audio_setting_input_gain | An integer 0 ~ 100 | Audio input gain |
| audio_setting_alarm_level | An integer 0 ~ 100 | Audio alarm level |

| Parameter | Values | Description |
| --- | --- | --- |
| format | inf | Export INF format |

Example: Get other stream parameter

http://192.168.0.200/cgi-bin/operator/operator.cgi?action=get.video.advanced

Return: Other stream parameter

HTTP/1.1 200 OK

Cache-Control: no-store, no-cache, must-revalidate, post-check=0, pre-check=0, max-age=-1

Content-Type: text/html; charset=UTF-8

Transfer-Encoding: chunked

Date: Sun, 02 Jan 2000 02:45:25 GMT

Server: Boa/0.94.13

var system_deviceinformation_model = 'MS-C3351'; var rtsp_port = '554'; var system_misc_audiooutvolume = '-14'; var audio_setting_enable = '0'; var audio_setting_codec = '2'; var audio_setting_source = '0'; var imaging_settings_osd_enabled = '1'; var imaging_settings_osd_text_enabled = '1'; var imaging_settings_osd_text_string = 'text2'; var imaging_settings_osd_datetime_enabled = '1'; var system_deviceinformation_sensor = '3m_0331'; var system_deviceinformation_tvstandards = -1'; var model_hardware_mic = '1'; var model_hardware_linein = '0';

Example: Get other stream parameter

http://192.168.0.200/cgi-bin/operator/operator.cgi?action=get.video.advanced

Return: Other stream parameter with INF format

HTTP/1.1 200 OK

Cache-Control: no-store, no-cache, must-revalidate, post-check=0, pre-check=0, max-age=-1

Content-Type: text/plain; charset=UTF-8

Transfer-Encoding: chunked

Date: Sun, 02 Jan 2000 02:45:41 GMT

Server: Boa/0.94.13

[VIDEO_ADVANCED]

system_deviceinformation_model=MS-C3351

rtsp_port=555

system_misc_audiooutvolume=-14

audio_setting_enable=1

audio_setting_codec=2

audio_setting_source=0

imaging_settings_osd_enabled=1

imaging_settings_osd_text_enabled=1

imaging_settings_osd_text_string=test2

imaging_settings_osd_datetime_enabled=1

system_deviceinformation_sensor=3m_0331

system_deviceinformation_tvstandards=-1

model_hardware_mic=1

model_hardware_linein=0

- 2.2.2 set.video.advanced

To set other stream parameter

Usage:

http://<serverIP>/cgi-bin/operator/operator.cgi?action=set.video.advanced

Example: Set other stream parameter

http://192.168.0.200/cgi-bin/operator/operator.cgi?action=set.video.advanced&imaging_settings_osd_enabled=1&imaging_settings_osd_text_enabled=1&imaging_settings_osd_text_string=Bruce.milesight&imaging_settings_osd_datetime_enabled=1&system_misc_audiooutvolume=-8&audio_setting_source=0&audio_setting_codec=1&audio_setting_enabled=1&system_deviceinformation_tvstandards=0

- 2.3 CAMERA SETTINGS

- 2.3.1 get.camera.setting

To get camera parameters.

| Parameter | Values | Description |
| --- | --- | --- |
| imaging_settings_tvstandards | An integer -1~1 | Select video source standard. -1 : Auto 0 : NTSC 1 : PAL |
| imaging_settings_brightness | An unsigned integer 0~100 | Image brightness |
| imaging_settings_colorsaturation | An unsigned integer 0~100 | Image saturation |
| imaging_settings_hue | An unsigned integer 0~100 | Image Hue (not implemented) |
| imaging_settings_contrast | An unsigned integer 0~100 | Image contrast (50/60/70/80/90 for not AM1000 serial and not AM7000 serial) |
| imaging_settings_exposure_mode | An unsigned integer 0~2 | Exposure mode. 0 Auto, 1 PAL, 2 NTSC |
| imaging_settings_exposure_maxexposuretime | An unsigned integer 1~3 | Select max. exposure time. 1 1/30, 2 1/15, 3 1/6 |
| imaging_settings_exposure_maxgain | An unsigned integer 0~4 | AGC 0 is mean max level. (not implemented) |
| Imaging_settings_ircutfilter (not implemented) | An unsigned integer 0~6 | Select ICR/LEN control mode. 0 By light-sensor, 1 Night mode, 2 Day mode, 3 By Time, 4 By GPIN status, 5 By PIR & light-sensor, 6 By PIR. |
| Imaging_settings_whitebalance_mode (not implemented) | An unsigned integer 0~5 | Select white balance mode 0 Auto, 1 Hold, 2 Outdoor_1, 3 Outdoor_2, 4 Indoor |
| imaging_settings_mirror_horizontal | An unsigned integer 0,1 | Horizontal mirror image. 1: enable |
| imaging_settings_mirror_vertical | An unsigned integer 0,1 | Vertical flip image. 1: enable |
| imaging_settings_voutstandards(not implemented) | An unsigned integer 0,1 | Video output standard 0 NTSC, 1 PAL |
| imaging_settings_iricr_dayhour | An unsigned integer 0~23 | Day hour for ICR/LEN control |
| imaging_settings_iricr_dayminute | An unsigned integer 0~59 | Day minute for ICR/LEN control |
| imaging_settings_iricr_nighthour | An unsigned integer 0~23 | Night hour for ICR/LEN control |
| imaging_settings_iricr_nightminute | An unsigned integer 0~59 | Night minute for ICR/LEN control |
| imaging_settings_iricr_gpinstatus(not implemented) | An unsigned integer 0,1 | GPIN status for Day for ICR/LEN control 0: close, 1: open |
| imaging_settings_pir_duration (not implemented) | An unsigned integer 5~30 | PIR duration (sec) for ICR/LEN control |
| imaging_settings_pir_sensitivity (not implemented) | An unsigned integer 10~100 | PIR sensitivity for ICR/LEN control 每10為一個階級 |
| imaging_settings_aewindows_0 | An unsigned integer 0~15 | AE window X |
| imaging_settings_aewindows_1 | An unsigned integer 0~14 | AE window Y |
| imaging_settings_aewindows_2 | An unsigned integer 0~15 | AW window Width |
| imaging_settings_aewindows_3 | An unsigned integer 0~14 | AW window Height |

Usage:

http://<serverIP>/cgi-bin/operator/operator.cgi?action=get.camera.setting[&<parameter>=<value>…]

| Parameter | Values | Description |
| --- | --- | --- |
| Format | inf | Export INF format |

Example: Get camera parameter

http://192.168.0.200/cgi-bin/operator/operator.cgi?action=get.camera.setting

Return: Get camera parameter

HTTP/1.1 200 OK

Cache-Control: no-store, no-cache, must-revalidate, post-check=0, pre-check=0, max-age=-1

Content-Type: text/html; charset=UTF-8

Transfer-Encoding: chunked

Date: Sat, 01 Jan 2000 03:11:17 GMT

Server: Boa/0.94.13

var imaging_settings_tvstandards = '-1'; var imaging_settings_brightness = '50'; var imaging_settings_colorsaturation = '50'; var imaging_settings_hue = '50'; var imaging_settings_contrast = '50'; var imaging_settings_exposure_mode = '1'; var imaging_settings_whitebalance_mode = '3'; var imaging_settings_exposure_maxexposuretime = '3'; var imaging_settings_exposure_maxgain = '0'; var Imaging_settings_ircutfilter = '0'; var imaging_settings_led_mode = '0; var imaging_settings_mirror_horizontal = '0'; var imaging_settings_mirror_vertical = '0'; var imaging_settings_iricr_dayhour = '16'; var imaging_settings_iricr_dayminute = '0'; var imaging_settings_iricr_nighthour = '6'; var imaging_settings_iricr_nightminute = '0'; var imaging_settings_iricr_gpinstatus = '0'; var imaging_settings_iricr_gpinnumber = '0'; var imaging_settings_pir_duration = '0'; var imaging_settings_pir_sensitivity = '10'; var imaging_settings_aewindows_0 = '0'; var imaging_settings_aewindows_1 = '0'; var imaging_settings_aewindows_2 = '0'; var imaging_settings_aewindows_3 = '0'; var media_profile_url = 'main'; var media_profile_resolution_width = '1920'; var media_profile_resolution_height = '1080';

Example: Get camera parameter

http://192.168.0.200/cgi-bin/operator/operator.cgi?action=get.camera.setting&format=inf

Return: Get camera parameter with INF format

HTTP/1.1 200 OK

Cache-Control: no-store, no-cache, must-revalidate, post-check=0, pre-check=0, max-age=-1

Content-Type: text/plain; charset=UTF-8

Transfer-Encoding: chunked

Date: Sat, 01 Jan 2000 03:11:25 GMT

Server: Boa/0.94.13

[CAMERA_SETTING]

imaging_settings_tvstandards=-1

imaging_settings_brightness=50

imaging_settings_colorsaturation=50

imaging_settings_hue=50

imaging_settings_contrast=50

imaging_settings_exposure_mode=1

imaging_settings_whitebalance_mode=3

imaging_settings_exposure_maxexposuretime=3

imaging_settings_exposure_maxgain=0

Imaging_settings_ircutfilter=0

imaging_settings_led_mode=0

imaging_settings_mirror_horizontal=0

imaging_settings_mirror_vertical=0

imaging_settings_iricr_dayhour=16

imaging_settings_iricr_dayminute=0

imaging_settings_iricr_nighthour=6

imaging_settings_iricr_nightminute=0

imaging_settings_iricr_gpinstatus=0

imaging_settings_iricr_gpinnumber=0

imaging_settings_pir_duration=0

imaging_settings_pir_sensitivity=10

imaging_settings_aewindows_0=0

imaging_settings_aewindows_1=0

imaging_settings_aewindows_2=0

imaging_settings_aewindows_3=0

media_profile_url=main

media_profile_resolution_width=1920

media_profile_resolution_height=1080

- 2.3.2 set.camera.setting

To set camera parameters.

Example: Set brightness to 65

http://192.168.0.200/cgi-bin/operator/operator.cgi?action=set.camera.setting&imaging_settings_brightness=128&imaging_settings_colorsaturation=128&imaging_settings_contrast=128&imaging_settings_exposure_mode=1&imaging_setings_exposure_maxexposuretime=1&imaging_settings_mirror_horizontal=0&imaging_settings_mirror_vertical=0&imaging_settings_iricr_dayhour=16&imaging_settings_iricr_dayminute=0&imaging_settings_iricr_nighthour=6&imaging_settings_iricr_nightminute=0&imaging_settings_aewindows_1=0&imaging_settings_aewindows_2=0&imaging_settings_aewindows_3=0

- 2.3.3 set.camera.loaddefault

To set camera parameter to default value.

Example: Set brightness to 65

http://192.168.0.200/cgi-bin/operator/operator.cgi?action=set.camera.loaddefault

- 2.3.4 set.camera.settings.bwmode (not implemented)

To set camera B/W mode.

| Parameter | Values | Description |
| --- | --- | --- |
| imaging_settings_bwmode | An unsigned integer 0~2 | Select image Back/White mode. 0 : By ICR/LEN setting 1 : Back/While 2 : Color |

Example: Set image to back/while mode

http://192.168.0.200/cgi-bin/operator/operator.cgi?action=set.camera.settings.bwmode&imaging_settings_bwmode=1

- 2.3.5 set.camera.settings.maxgain  (not implemented)

To set camera gain for auto mode.

| Parameter | Values | Description |
| --- | --- | --- |
| imaging_settings_autogain_mode | An unsigned integer 0~1 | Enable auto gain mode |
| imaging_settings_autogain_day | An unsigned integer 0~5 | Max.gain for Day mode |
| imaging_settings_autogain_night | An unsigned integer 0~5 | Max gain for Night mode. |
| imaging_settings_exposure_maxgain | An unsigned integer 0~5 | See 2.3.2 |

Example: Set max. gain by ICR/LED control

http://192.168.0.200/cgi-bin/operator/operator.cgi?action=set.camera.settings.maxgain&imaging_settings_autogain_mode=1&imaging_settings_autogain_day=3&imaging_settings_autogain_night=1&barcmd=9

- 2.4 CAMERA PTZ

- 2.4.1 get.camera.ptz

To get PTZ/Serial port parameter

| Parameter | Values | Description |
| --- | --- | --- |
| ptz_setting_enable (not implemented) | An unsigned integer 0~1 | Enable PTZ control. (Fixed Value 1) 0 : No 1 : Yes |
| ptz_setting_protocol | An unsigned intege 0~3 | Select PTZ protocol 0 : PelcoD 1 : PelXXX 2 : Lilin 3 : DynaColor |
| ptz_setting_deviceid (not implemented) | An integer | PTZ device ID (Fixed Value 0) |
| ptz_setting_speed | An unsigned intege 1~10 | PTZ speed |
| ptz_setting_channel (not implemented) | An unsigned intege 0~3 | PTZ mapping to video-in (Fixed Value 0) |
| system_serial_buadrate | An unsigned intege 300,1200,2400, 4800,9600, 19200,38400, 57500,115200 | Baud rate for serial port |
| system_serial_databit (not implemented) | An unsigned intege 7,8 | Data bits for serial port |
| system_serial_stopbit (not implemented) | An unsigned intege 0,1 | Stop bits for serial port |
| system_serial_parity (not implemented) | An unsigned intege 0~2 | Parity for serial port 0 None, |
| system_serial_flowcontrol (not implemented) | An unsigned intege 0~2 | Flow control for serial port |

| Parameter | Values | Description |
| --- | --- | --- |
| Format | inf | Export INF format |

Example: Get PTZ/Serial port parameter

http://192.168.0.200 /cgi-bin/operator/operator.cgi?action=get.camera.ptz

only channel 1

Return:

HTTP/1.1 200 OK

Cache-Control: no-store, no-cache, must-revalidate, post-check=0, pre-check=0, max-age=-1

Content-Type: text/html; charset=UTF-8

Transfer-Encoding: chunked

Date: Mon, 03 Jan 2000 03:48:03 GMT

Server: Boa/0.94.13

ptz_setting_channel[0] = '0'; ptz_setting_enable[0] = '1'; ptz_setting_protocol[0] = '0'; ptz_setting_deviceid[0] = '0'; ptz_setting_speed[0] = '4'; var system_serial_buadrate = '4';

Example: Get PTZ/Serial port parameter

http://192.168.0.200 /cgi-bin/operator/operator.cgi?action=get.camera.ptz&format=inf

Return: INF format

HTTP/1.1 200 OK

Cache-Control: no-store, no-cache, must-revalidate, post-check=0, pre-check=0, max-age=-1

Content-Type: text/plain; charset=UTF-8

Transfer-Encoding: chunked

Date: Sat, 01 Jan 2000 03:40:25 GMT

Server: Boa/0.94.13

[CAMERA_PTZ]

ptz_setting_channel_0=0

ptz_setting_enable_0=1

ptz_setting_protocol_0=0

ptz_setting_deviceid_0=0

ptz_setting_speed_0=4

system_serial_buadrate=4

- 2.4.2 set.camera.ptz

To set PTZ/Serial port parameter

Example: set the PTZ enable(ID:2/speed:5/protocol:PelXXX) and Serial port (burate:2400/data:8/stop:1/parity:None) of server.

http://192.168.0.200/cgi-bin/operator/operator.cgi?action=set.camera.ptz&ptz_setting_protocol=0&ptz_setting_speed=5&system_serial_buadrate=2400

- 2.5 STORAGE DISK INFORMATION

- 2.5.1 get.recordfile.format (not implemented)

To get record information

| Parameter | Values | Description |
| --- | --- | --- |
| system_storage_information_recordfile | String Mp4, file | Select Record file format |
| system_storage_information_rec_channel | An unsigned integer 0,2,4,(6) | Select Record channel |
| system_storage_information_duration | An unsigned integer 10~900 | Record file duration |
| system_storage_information_prealarm_enable | An unsigned integer 0, 1 | Enable pre-alarm record |
| system_storage_information_prealarm_second | An unsigned integer 1~3 | Select pre-alarm second |

| Parameter | Values | Description |
| --- | --- | --- |
| format | inf | Export INF format |

Example: Get record infomation

http://192.168.0.200/cgi-bin/operator/operator.cgi?action=get.recordfile.format

Return:

HTTP/1.1 200 OK

Cache-Control: no-store, no-cache, must-revalidate, post-check=0, pre-check=0, max-age=-1

Content-Type: text/html; charset=UTF-8

Transfer-Encoding: chunked

Date: Sat, 01 Jan 2000 03:52:16 GMT

Server: Boa/0.94.13

var system_storage_information_recordfile = 'mp4'; var system_storage_information_rec_channel = '0'; var system_storage_information_duration = '900'; var system_storage_information_prealarm_enable = '0'; var system_storage_information_prealarm_second = '2';media_profile_id[0] = '0'; media_profile_url[0] = 'v01';media_profile_id[1] = '2'; media_profile_url[1] = 'v03';media_profile_id[2] = '4'; media_profile_url[2] = 'v05';media_profile_id[3] = '6'; media_profile_url[3] = 'v07';var system_deviceinformation_sensor = '2m';

Example: Get record infomation

http://192.168.0.200/cgi-bin/operator/operator.cgi?action=get.recordfile.format&format=inf

Return:

HTTP/1.1 200 OK

Cache-Control: no-store, no-cache, must-revalidate, post-check=0, pre-check=0, max-age=-1

Content-Type: text/plain; charset=UTF-8

Transfer-Encoding: chunked

Date: Sat, 01 Jan 2000 03:52:22 GMT

Server: Boa/0.94.13

[RECORDFILE_FORMAT]

system_storage_information_recordfile=mp4

system_storage_information_rec_channel=0

system_storage_information_duration=900

system_storage_information_prealarm_enable

system_storage_information_prealarm_second

media_profile_id_0=0

media_profile_url_0=v01

media_profile_id_1=2

media_profile_url_1=v03

media_profile_id_2=4

media_profile_url_2=v05

media_profile_id_3=6

media_profile_url_3=v07

system_deviceinformation_sensor=2m

- 2.5.2 set.recordfile.format (not implemented)

To set the format of the record files.

Example: Set record file format

http://192.168.0.200/cgi-bin/operator/operator.cgi?action=set.recordfile.format&system_storage_information_recordfile=mp4&system_storage_information_rec_channel=0&system_storage_information_prealarm_enable=0&system_storage_information_prealarm_second=2&system_storage_information_duration=900

- 2.5.3 set.storage.format (not implemented)

Format SD Card

Example: Format SD card

http://192.168.0.200/cgi-bin/operator/operator.cgi?action=set.storage.format

- 2.5.4 record.start (not implemented)

Start recording

Example: Start recording

http://192.168.0.200/cgi-bin/operator/operator.cgi?action= record.start

- 2.5.5 record.stop (not implemented)

Stop recording

Example: Stop recording

http://192.168.0.200/cgi-bin/operator/operator.cgi?action= record.stop

- 2.5.6 record.continue (not implemented)

Stop current record file and start a new one.

http://<serverIP>/cgi-bin/operator/operator.cgi?action=record.continue

Example: Stop current record file and start a new one.

http://192.168.0.200/cgi-bin/operator/operator.cgi?action=record.continue

- 2.6 EVENT CONFIGURATION

- 2.6.1 get.event.configuration

To get all event rule configuration

| Parameter | Values | Description |
| --- | --- | --- |
| event_rule_name (not implemented) | String, max 20 | The alias of an event rule |
| event_rule_enabled | Boolean | Enable the event rule 0 : No 1 : Yes |
| event_rule_periodic | Boolean | Event rule type 0 : Always 1 : Schedule |
| event_rule_weekdays (not implemented) | String, max 7 | Format : xxxxxx 1000000 : Sunday is set 1000001 : Sunday and Saturday are set. |
| event_rule_starttime (not implemented) | An unsigned integer 0~3600x24 | When periodic is 1, the start time. |
| event_rule_duration (not implemented) | An unsigned integer 0~3600x24 | When periodic is 1, the end time. |
| event_rule_triggerbymotion_enable | Boolean | Enable Trigger by motion detection. 0 : No 1 : Yes |
| event_rule_triggerbymotion_window_id (not implemented) | An unsigned integer | Motion detection windows ID. |
| event_rule_triggerbyboot_enable | Boolean | Enable trigger when device boots. 0 : No 1 : Yes |
| event_rule_triggerbyschedule_enable | Boolean | Enable trigger by schedule. 0 : No 1 : Yes |
| event_rule_actionupload_enabled | Boolean | Enable upload action. 0 : No 1 : Yes |
| event_rule_actionupload_type | 0 | Fixed value. ( Upload via FTP ) |
| event_rule_actionupload_ftp_id | An unsigned integer | FTP server id. |
| event_rule_actionupload_file_type | 0~2 | 0:for avi 1:for jpg 2:avi & jpg |
| event_rule_actionupload_post_alarm | An unsigned integer 1~200 | When trigger is not by ‘Schedule’. |
| event_rule_actionupload_interval | An unsigned integer 1~300000 | 1.1 seconds = 1100. |
| event_rule_actionemail_enabled | Boolean | Enable email action. 0 : No 1 : Yes |
| event_rule_actionemail_smtp_id | An unsigned integer | SMTP server id. |
| event_rule_actionemail_file_type | An unsigned integer | 0:JPEG 1:AVI |
| event_rule_actionemail_post_alarm | An unsigned integer 1~200 | When trigger is not by ‘Schedule’. |
| event_rule_actionemail_interval | An unsigned integer 1~300000 | 1.1 seconds = 1100. |
| event_rule_actionemail_smtp_email | String, max 80 | Target email address. |
| event_rule_actionemail_smtp_subject | String, max 80 | Email subject. |
| event_rule_actionemail_smtp_information | String, max 80 | Email message. |
| event_rule_actionemail_smtp_upload | Boolean | Enable upload file. 0 : No 1 : Yes |
| event_rule_actionhttp_enabled | Boolean | Enable HTTP action. 0 : No 1 : Yes |
| event_rule_actionhttp_http_id (not implemented) | An unsigned integer | HTTP server id. (not implemented) |
| event_rule_actionhttp_http_post_alarm (not implemented) | An unsigned integer 1~200 | When trigger is not by ‘Schedule’. |
| event_rule_actionhttp_http_interval (not implemented) | An unsigned integer 1~300000 | 1.1 seconds = 1100. |
| event_rule_actionhttp_http_message (not implemented) | String, max 80 | HTTP message. |
| event_rule_actiontcp_enabled (not implemented) | Boolean | Enable TCP action. |
| event_rule_actiontcp_tcp_id (not implemented) | An unsigned integer | TCP server id. (not implemented) |
| event_rule_actiontcp_tcp_post_alarm (not implemented) | An unsigned integer 1~200 | When trigger is not by ‘Schedule’. |
| event_rule_actiontcp_tcp_interval (not implemented) | An unsigned integer 1~300000 | 1.1 seconds = 1100. |
| event_rule_actiontcp_tcp_message (not implemented) | String, max 80 | TCP message. |
| event_rule_actionrecord_enabled | Boolean | Enable record to SD. 0 : No 1 : Yes |
| event_rule_triggerbygpin_enable | Boolean | 0 : No 1 : Yes |
| event_rule_triggerbygpin_id | An unsigned integer | GPIN id. |
| event_rule_actiongpout_enabled | Boolean | 0 : No 1 : Yes |
| event_rule_actiongpout_id | An unsigned integer | GPOUT id. |
| event_rule_id | An unsigned integer | The rule index id. |
| event_rule_triggerbygpin_idlestate | Boolean | Normal state for gio input 0:Low level 1:High level |

| Parameter | Values | Description |
| --- | --- | --- |
| format | inf | Export INF format |

Example: Get all event ruler

http://192.168.0.200/cgi-bin/operator/operator.cgi?action=get.event.configuration

Return: No event rule configuration

HTTP/1.1 200 OK

Cache-Control: no-store, no-cache, must-revalidate, post-check=0, pre-check=0, max-age=-1

Content-Type: text/html; charset=UTF-8

Transfer-Encoding: chunked

Date: Sat, 01 Jan 2000 04:57:41 GMT

Server: Boa/0.94.13

var pir_record_action = '0';var model_hardware_gpin = '1'; var model_hardware_gpout = '1';io_relayoutput_token[0] = '1'; io_relayoutput_properties_mode[0] = '0'; io_relayoutput_properties_delaytime[0] = 5; io_relayoutput_properties_idlestate[0] = '1'; io_relayoutput_logicalstate[0] = '1'; io_relayoutput_io_type[0] = '1';io_relayoutput_token[1] = '1'; io_relayoutput_properties_mode[1] = '0'; io_relayoutput_properties_delaytime[1] = 5; io_relayoutput_properties_idlestate[1] = '1'; io_relayoutput_logicalstate[1] = '1'; io_relayoutput_io_type[1] = '0';var system_io = 1;var system_deviceinformation_model = 'AM4210';

Example: Get all event ruler

http://192.168.0.200/cgi-bin/operator/operator.cgi?action=get.event.configuration

Return: With event rule configuration

HTTP/1.1 200 OK

Cache-Control: no-store, no-cache, must-revalidate, post-check=0, pre-check=0, max-age=-1

Content-Type: text/html; charset=UTF-8

Transfer-Encoding: chunked

Date: Sat, 01 Jan 2000 05:19:18 GMT

Server: Boa/0.94.13

event_rule_id[0] = '0'; event_rule_name[0] = 'msevent'; event_rule_enabled[0] = '1'; event_rule_periodic[0] = '0'; event_rule_triggerbymotion_enable[0] = '0'; event_rule_triggerbyboot_enable[0] = '0'; event_rule_triggerbyschedule_enable[0] = '0'; event_rule_triggerbygpin_enable[0] = '0'; event_rule_actionupload_enabled[0] = '0'; event_rule_actionemail_enabled[0] = '0'; event_rule_actionhttp_enabled[0] = '0'; event_rule_actiontcp_enabled[0] = '0'; event_rule_actionrecord_enabled[0] = '0'; event_rule_actiongpout_enabled[0] = '0'; event_rule_weekdays[0] = '1111111'; event_rule_starttime[0] = '0'; event_rule_duration[0] = '0'; event_rule_triggerbymotion_window_id[0] = '0'; event_rule_actionupload_type[0] = '0'; event_rule_actionupload_ftp_id[0] = '0'; event_rule_actionupload_post_alarm[0] = '10'; event_rule_actionupload_interval[0] = '30000'; event_rule_actionupload_file_type[0] = '0'; event_rule_actionemail_smtp_id[0] = '0'; event_rule_actionemail_smtp_email[0] = 'user@domain.com'; event_rule_actionemail_smtp_subject[0] = 'MS'; event_rule_actionemail_smtp_information[0] = 'MS mail.'; event_rule_actionemail_smtp_upload[0] = '1'; event_rule_actionemail_post_alarm[0] = '10'; event_rule_actionemail_interval[0] = '30000'; event_rule_actionemail_file_type[0] = '0'; event_rule_actionhttp_http_id[0] = '0'; event_rule_actionhttp_http_post_alarm[0] = '10'; event_rule_actionhttp_http_interval[0] = '1000'; event_rule_actionhttp_http_parameter[0] = ''; event_rule_actionhttp_http_message[0] = ''; event_rule_actiontcp_tcp_id[0] = '1'; event_rule_actiontcp_tcp_post_alarm[0] = '10'; event_rule_actiontcp_tcp_interval[0] = '1000'; event_rule_actiontcp_tcp_message[0] = ''; event_rule_triggerbygpin_id[0] = '1'; event_rule_actiongpout_id[0] = '1'; var pir_record_action = '0'; var model_hardware_gpin = '1'; var model_hardware_gpout = '1'; var system_io = '1'; var system_deviceinformation_model = 'MS-C3351';

Example: Get all event rule

http://192.168.0.200/cgi-bin/operator/operator.cgi?action=get.event.configuration&format=inf

Return: With event rule configuration

HTTP/1.1 200 OK

Cache-Control: no-store, no-cache, must-revalidate, post-check=0, pre-check=0, max-age=-1

Content-Type: text/plain; charset=UTF-8

Transfer-Encoding: chunked

Date: Sat, 01 Jan 2000 05:19:21 GMT

Server: Boa/0.94.13

[EVENT_CONFIGURATION]

event_rule_id_0=0

event_rule_name_0=mseventevent_rule_enabled_0=1

event_rule_periodic_0=0

event_rule_triggerbymotion_enable_0=0

event_rule_triggerbyboot_enable_0=0

event_rule_triggerbyschedule_enable_0=0

event_rule_triggerbygpin_enable_0=0

event_rule_actionupload_enabled_0=0

event_rule_actionemail_enabled_0=0

event_rule_actionhttp_enabled_0=0

event_rule_actiontcp_enabled_0=0

event_rule_actionrecord_enabled_0=0

event_rule_actiongpout_enabled_0=0

event_rule_weekdays_0=1111111

event_rule_starttime_0=0

event_rule_duration_0=0

event_rule_triggerbymotion_window_id_0=0

event_rule_actionupload_type_0=0

event_rule_actionupload_ftp_id_0=0

event_rule_actionupload_post_alarm_0=10

event_rule_actionupload_interval_0=30000

event_rule_actionupload_file_type_0=0

event_rule_actionemail_smtp_id_0=0

event_rule_actionemail_smtp_email_0=user@domain.com

event_rule_actionemail_smtp_subject_0=MS

event_rule_actionemail_smtp_information_0=MS mail.

event_rule_actionemail_smtp_upload_0=1

event_rule_actionemail_post_alarm_0=10

event_rule_actionemail_interval_0=30000

event_rule_actionemail_file_type_0=0

event_rule_actionhttp_http_id_0=0

event_rule_actionhttp_http_post_alarm_0=10

event_rule_actionhttp_http_interval_0=1000

event_rule_actionhttp_http_parameter_0=

event_rule_actionhttp_http_message_0=

event_rule_actiontcp_tcp_id_0=1

event_rule_actiontcp_tcp_post_alarm_0=10

event_rule_actiontcp_tcp_interval_0=1000

event_rule_actiontcp_tcp_message_0=

event_rule_triggerbygpin_id_0=1

event_rule_actiongpout_id_0=1

var pir_record_action=0

var model_hardware_gpin=1

var model_hardware_gpout=1

var system_io=1

var system_deviceinformation_model=MS-C3351

Example: Get all event ruler

http://192.168.0.200/cgi-bin/operator/operator.cgi?action=get.event.configuration&format=inf

Return: No event rule configuration

HTTP/1.1 200 OK

Cache-Control: no-store, no-cache, must-revalidate, post-check=0, pre-check=0, max-age=-1

Content-Type: text/plain; charset=UTF-8

Transfer-Encoding: chunked

Date: Sat, 01 Jan 2000 04:57:48 GMT

Server: Boa/0.94.13

[EVENT_CONFIGURATION]

var pir_record_action=0

var model_hardware_gpin=1

var model_hardware_gpout=1

var system_io=1

var system_deviceinformation_model=MS-C3351

- 2.6.2 update.event.configuration

Update Event rule configuration

Example: Modify schedule recording event rule with GPOUT

http://192.168.0.200/cgi-bin/operator/operator.cgi?action=update.event.configuration&event_rule_enabled=1&event_rule_triggerbymotion_enable=0&event_rule_actionupload_enabled=0&event_rule_actionupload_type=0&event_rule_actionupload_ftp_id=0&event_rule_actionupload_file_type=0&event_rule_actionupload_post_alarm=10&event_rule_actionupload_interval=1000&event_rule_actionemail_enabled=0&event_rule_actionemail_file_type=0&event_rule_actionemail_post_alarm=10&event_rule_actionemail_interval=1000&event_rule_actionemail_smtp_email=&event_rule_actionemail_smtp_subject=&event_rule_actionemail_smtp_information=&event_rule_actionemail_smtp_upload=0&event_rule_actionrecord_enabled=1&event_rule_triggerbygpin_enable=0&event_rule_triggerbygpin_id=1&event_rule_actiongpout_enabled=1&event_rule_actiongpout_id=1

- 2.6.3 add.event.configuration

Add event rule

http://<serverIP>/cgi-bin/operator/operator.cgi?action=add.event.configuration[&<parameter>=<value>…]

Note : event_rule_id will be created by system automatticly.

Example: Add schedule recording event rule

http://192.168.0.200/cgi-bin/operator/operator.cgi?action=add.event.configuration&event_rule_id=&event_rule_name=tset&event_rule_enabled=1&event_rule_periodic=0&event_rule_weekdays=0000000&event_rule_starttime=0&event_rule_duration=0&event_rule_triggerbymotion_enable=0&event_rule_triggerbymotion_window_id=0&event_rule_triggerbyboot_enable=0&event_rule_triggerbyschedule_enable=1&event_rule_actionupload_enabled=0&event_rule_actionupload_type=0&event_rule_actionupload_ftp_id=0&event_rule_actionupload_file_type=0&event_rule_actionupload_post_alarm=10&event_rule_actionupload_interval=1000&event_rule_actionemail_enabled=0&event_rule_actionemail_smtp_id=0&event_rule_actionemail_file_type=0&event_rule_actionemail_post_alarm=10&event_rule_actionemail_interval=1000&event_rule_actionemail_smtp_email=&event_rule_actionemail_smtp_subject=&event_rule_actionemail_smtp_information=&event_rule_actionemail_smtp_upload=0&event_rule_actionhttp_enabled=0&event_rule_actionhttp_http_id=0&event_rule_actionhttp_http_post_alarm=10&event_rule_actionhttp_http_interval=1000&event_rule_actionhttp_http_parameter=&event_rule_actionhttp_http_message=&event_rule_actiontcp_enabled=0&event_rule_actiontcp_tcp_id=0&event_rule_actiontcp_tcp_post_alarm=10&event_rule_actiontcp_tcp_interval=1000&event_rule_actiontcp_tcp_message=&event_rule_actionrecord_enabled=1&event_rule_triggerbygpin_enable=0&event_rule_triggerbygpin_id=1&event_rule_actiongpout_enabled=0&event_rule_actiongpout_id=0

- 2.6.4 del.event.configuration

Delete event rule by ID

| Parameter | Values | Description |
| --- | --- | --- |
| event_rule_id | integer | Delete event rule ID |

Example: Get all event server

http://192.168.0.200/cgi-bin/operator/operator.cgi?action=del.event.configuration

- 2.7 EVENT MOTION DETECTION

- 2.7.1 get.event.motion.detection

To get motion window configuration.

| Parameter | Values | Description |
| --- | --- | --- |
| motion_window_name (not implemented) | String, max 20 | Motion detection window nickname |
| motion_window_color (not implemented) | An unsigned integer 0~2 | Motion detection window display color 0 : Red 1 : Green 2 : Blue |
| motion_window_rectangle_x | An unsigned integer | Top left X coordinates |
| motion_window_rectangle_y | An unsigned integer | Top left Y coordinates |
| motion_window_rectangle_width | An unsigned integer | Motion detection window width |
| motion_window_rectangle_height | An unsigned integer | Motion detection window height |
| motion_window_trigger_sensitivity | An unsigned integer 1~10 | Motion sensitivity. |
| motion_window_id | An unsigned integer 0~12 | Motion detection window ID |

| Parameter | Values | Description |
| --- | --- | --- |
| format | inf | Export INF format |

Example: Get motion window configuration

http://192.168.0.200/cgi-bin/operator/operator.cgi?action=get.event.motion.detection

Return: No motion detection configuration

HTTP/1.1 200 OK

Cache-Control: no-store, no-cache, must-revalidate, post-check=0, pre-check=0, max-age=-1

Content-Type: text/html; charset=UTF-8

Transfer-Encoding: chunked

Date: Sat, 01 Jan 2000 19:49:01 GMT

Server: Boa/0.94.13

var rtsp_port = '554'; var media_profile_url = 'main'; var media_profile_resolution_width = '1920'; var media_profile_resolution_height = '1080'; var imaging_settings_osd_test = '0'; var system_io = '1'; var system_deviceinformation_model = 'MS-C3351';

Return: With motion detection block configuration

HTTP/1.1 200 OK

Cache-Control: no-store, no-cache, must-revalidate, post-check=0, pre-check=0, max-age=-1

Content-Type: text/html; charset=UTF-8

Transfer-Encoding: chunked

Date: Sat, 01 Jan 2000 20:10:36 GMT

Server: Boa/0.94.13

var rtsp_port = '554'; var media_profile_url = 'main'; var media_profile_resolution_width = '1920'; var media_profile_resolution_height = '1080'; var imaging_settings_osd_test = '0'; var system_io = '1'; motion_window_id[0] = '0'; motion_window_name[0] = 'msmotion0'; motion_window_color[0] = '0'; motion_window_rectangle_x[0] = '0'; motion_window_rectangle_y[0] = '360'; motion_window_rectangle_width[0] = '480'; motion_window_rectangle_height[0] = '360'; motion_window_trigger_sensitivity[0] = '50'; motion_window_id[1] = '1'; motion_window_name[1] = 'msmotion1'; motion_window_color[1] = '0'; motion_window_rectangle_x[1] = '480'; motion_window_rectangle_y[1] = '360'; motion_window_rectangle_width[1] = '480'; motion_window_rectangle_height[1] = '360'; motion_window_trigger_sensitivity[1] = '50'; motion_window_id[2] = '2'; motion_window_name[2] = 'msmotion2'; motion_window_color[2] = '0'; motion_window_rectangle_x[2] = '960'; motion_window_rectangle_y[2] = '360'; motion_window_rectangle_width[2] = '480'; motion_window_rectangle_height[2] = '360'; motion_window_trigger_sensitivity[2] = '50'; motion_window_id[3] = '3'; motion_window_name[3] = 'msmotion3'; motion_window_color[3] = '0'; motion_window_rectangle_x[3] = '480'; motion_window_rectangle_y[3] = '0'; motion_window_rectangle_width[3] = '480'; motion_window_rectangle_height[3] = '360'; motion_window_trigger_sensitivity[3] = '50'; motion_window_id[4] = '4'; motion_window_name[4] = 'msmotion4'; motion_window_color[4] = '0'; motion_window_rectangle_x[4] = '960'; motion_window_rectangle_y[4] = '0'; motion_window_rectangle_width[4] = '480'; motion_window_rectangle_height[4] = '360'; motion_window_trigger_sensitivity[4] = '50'; motion_window_id[5] = '5'; motion_window_name[5] = 'msmotion5'; motion_window_color[5] = '0'; motion_window_rectangle_x[5] = '1440'; motion_window_rectangle_y[5] = '0'; motion_window_rectangle_width[5] = '480'; motion_window_rectangle_height[5] = '360'; motion_window_trigger_sensitivity[5] = '50'; var system_deviceinformation_model = 'MS-C3351'

Example: Get motion window configuration

http://192.168.0.200/cgi-bin/operator/operator.cgi?action=get.event.motion.detection&format=inf

Return: No motion detection configuration

HTTP/1.1 200 OK

Cache-Control: no-store, no-cache, must-revalidate, post-check=0, pre-check=0, max-age=-1

Content-Type: text/plain; charset=UTF-8

Transfer-Encoding: chunked

Date: Sat, 01 Jan 2000 19:49:10 GMT

Server: Boa/0.94.13

[EVENT_MOTION_DETECTION]

rtsp_port=554

media_profile_url=main

media_profile_resolution_width=1920

media_profile_resolution_height=1080

imaging_settings_osd_test=0

system_io=1

system_deviceinformation_model=MS-C3351

Return: With motion detection block configuration

HTTP/1.1 200 OK

Cache-Control: no-store, no-cache, must-revalidate, post-check=0, pre-check=0, max-age=-1

Content-Type: text/plain; charset=UTF-8

Transfer-Encoding: chunked

Date: Sat, 01 Jan 2000 20:10:39 GMT

Server: Boa/0.94.13

[EVENT_MOTION_DETECTION]

rtsp_port=554

media_profile_url=main

media_profile_resolution_width=1920

media_profile_resolution_height=1080

imaging_settings_osd_test=0

system_io=1

motion_window_id_0=0

motion_window_name_0=msmotion0

motion_window_color_0=0

motion_window_rectangle_x_0=0

motion_window_rectangle_y_0=360

motion_window_rectangle_width_0=480

motion_window_rectangle_height_0=360

motion_window_trigger_sensitivity_0=50

motion_window_id_1=1

motion_window_name_1=msmotion1

motion_window_color_1=0

motion_window_rectangle_x_1=480

motion_window_rectangle_y_1=360

motion_window_rectangle_width_1=480

motion_window_rectangle_height_1=360

motion_window_trigger_sensitivity_1=50

motion_window_id_2=2

motion_window_name_2=msmotion2

motion_window_color_2=0

motion_window_rectangle_x_2=960

motion_window_rectangle_y_2=360

motion_window_rectangle_width_2=480

motion_window_rectangle_height_2=360

motion_window_trigger_sensitivity_2=50

motion_window_id_3=3

motion_window_name_3=msmotion3

motion_window_color_3=0

motion_window_rectangle_x_3=480

motion_window_rectangle_y_3=0

motion_window_rectangle_width_3=480

motion_window_rectangle_height_3=360

motion_window_trigger_sensitivity_3=50

motion_window_id_4=4

motion_window_name_4=msmotion4

motion_window_color_4=0

motion_window_rectangle_x_4=960

motion_window_rectangle_y_4=0

motion_window_rectangle_width_4=480

motion_window_rectangle_height_4=360

motion_window_trigger_sensitivity_4=50

motion_window_id_5=5

motion_window_name_5=msmotion5

motion_window_color_5=0

motion_window_rectangle_x_5=1440

motion_window_rectangle_y_5=0

motion_window_rectangle_width_5=480

motion_window_rectangle_height_5=360

motion_window_trigger_sensitivity_5=50

system_deviceinformation_model=MS-C3351

- 2.7.2 set.motion.osd (not implemented)

Enable motion test OSD

http://<serverIP>/cgi-bin/operator/operator.cgi?action=set.motion.osd[&<parameter>=<value>…]

| Parameter | Values | Description |
| --- | --- | --- |
| imaging_settings_osd_test | An unsigned integer 0,1 | Enable motion detection OSD |

Example: Enable motion detection OSD

http://192.168.0.200/cgi-bin/operator/operator.cgi?action=set.motion.osd&imaging_settings_osd_test=1

- 2.7.3 update.event.motion.detection

Update motion detection windows configuration

http://<serverIP>/cgi-bin/operator/operator.cgi?action=update.event.motion.detection[&<parameter>=<value>…]

Example: Modify motion detection window configuration

http://192.168.0.200/cgi-bin/operator/operator.cgi?action=update.event.motion.detection&motion_window_trigger_sensitivity=5&motion_window_rectangle_x=0&motion_window_rectangle_y=0&motion_window_rectangle_width=1280&motion_window_rectangle_height=720&motion_window_id=0

- 2.7.4 add.event.motion.detection

Add motion detection windows configurations

http://<serverIP>/cgi-bin/operator/operator.cgi?action=add.event.motion.detection[&<parameter>=<value>…]

Example: Add motion detection window by ID

http://192.168.0.200/cgi-bin/operator/operator.cgi?action=add.event.motion.detection&motion_window_trigger_sensitivity=5&motion_window_rectangle_x=0&motion_window_rectangle_y=0&motion_window_rectangle_width=1280&motion_window_rectangle_height=720&motion_window_id=0

- 2.7.5 del.event.motion.detection

Delete motion detection configuration by ID

http://<serverIP>/cgi-bin/operator/operator.cgi?action=del.event.configuration[&<parameter>=<value>…]

| Parameter | Values | Description |
| --- | --- | --- |
| motion_window_id | An unsigned integer 0~2 | Delete motion detection windows ID |

Example: Delete motion detection window by ID

http://192.168.0.200/cgi-bin/operator/operator.cgi?action=del.event.motion.detection&motion_window_id=0

- 2.8 EVENT SERVER

- 2.8.1 get.event.server

Get all server configuration for event rule

| Parameter | Values | Description |
| --- | --- | --- |
| server_ftp_name | String, max 32 | Nickname for FTP server |
| server_ftp_address | IP address&域名 | FTP server address |
| server_ftp_port | Network port | FTP port |
| server_ftp_path | String, max 50 | FTP upload path |
| server_ftp_username | String, max 32 | FTP username |
| server_ftp_password | String, max 32 | FTP password |
| server_ftp_id | An unsigned integer | FTP server ID (Fixed value 0) |

| server_http_name | String, max 32 | Nickname for HTTP server |
| --- | --- | --- |
| server_http_url | String, max 256 | HTTP server address(support url encode: http://192.168.5.100/notify.cgi change to http%3A%2F%2F192.168.5.100%2Fnotify.cgi) |
| server_http_username | String, max 32 | HTTP username |
| server_http_password | String, max 16 | HTTP password |
| server_http_id | An unsigned integer | HTTP server ID FOR [MSTI]: 0:motion http server 1:external input http server FOR [MSAC&MSHC]: 0:motion http server 1:network lost server 2:audio alarm in http server 3:extenal input alarm http server 4:tamper alarm http server 5:PIR alarm http server 6:ip conflicted alarm |

| - server_tcp_name(not implemented) | String, max 32 | Nickname for TCP server |
| --- | --- | --- |
| server_tcp_address(not implemented) | IP address | TCP server address |
| server_tcp_port(not implemented) | Network port | TCP port |
| server_tcp_id(not implemented) | An unsigned integer | TCP server ID |

| server_smtp_name | String, max 32 | Nickname for SMTP server |
| --- | --- | --- |
| server_smtp_address | IP address | SMTP server address |
| server_smtp_port | An unsigned integer | SMTP port |
| server_smtp_username | String, max 32 | Email username |
| server_smtp_password | String, max 32 | Email password |
| server_smtp_ssl | Boolean | Enable login with SSL 0 : No 1 : YES |
| server_smtp_tls | Boolean | Enable login with TLS 0 : No 1 : YES |
| server_smtp_from_address | String, max 80 | Email address |
| server_smtp_id | An unsigned integer | SMTP server ID (Fixed value 0) |
| server_smtp_to_address | String, max 80 | Receiver email address. |

| Parameter | Values | Description |
| --- | --- | --- |
| format | inf | Export INF format |

Example: Get all event server

http://192.168.0.200/cgi-bin/operator/operator.cgi?action=get.event.server

Return: No event server

HTTP/1.1 200 OK

Cache-Control: no-store, no-cache, must-revalidate, post-check=0, pre-check=0, max-age=-1

Content-Type: text/html; charset=UTF-8

Transfer-Encoding: chunked

Date: Sat, 01 Jan 2000 20:37:37 GMT

Server: Boa/0.94.13

server_ftp_id[0] = '0'; server_ftp_name[0] = 'msftp0'; server_ftp_address[0] = '192.168.5.1'; server_ftp_path[0] = 'default_folder'; server_ftp_username[0] = 'ftpuser'; server_ftp_password[0] = '9999'; server_smtp_name[0] = 'mssmtp0'; server_smtp_address[0] = 'smtp.sina.com'; server_smtp_port[0] = '25'; server_smtp_username[0] = 'hdipnc'; server_smtp_password[0] = 'hdipnc2013ms'; server_smtp_ssl[0] = '0'; server_smtp_from_address[0] = 'hdipnc@sina.com'; server_smtp_id[0] = '0'; var system_deviceinformation_model = 'MS-C3351';

Return: with event server

HTTP/1.1 200 OK

Cache-Control: no-store, no-cache, must-revalidate, post-check=0, pre-check=0, max-age=-1

Content-Type: text/html; charset=UTF-8

Transfer-Encoding: chunked

Date: Sat, 01 Jan 2000 21:00:24 GMT

Server: Boa/0.94.13

server_ftp_id[0] = '0'; server_ftp_name[0] = 'msftp0'; server_ftp_address[0] = '192.168.5.1'; server_ftp_path[0] = 'default_folder'; server_ftp_username[0] = 'ftpuser'; server_ftp_password[0] = '9999'; server_smtp_name[0] = 'mssmtp0'; server_smtp_address[0] = 'smtp.sina.com'; server_smtp_port[0] = '25'; server_smtp_username[0] = 'hdipnc'; server_smtp_password[0] = 'hdipnc2013ms'; server_smtp_ssl[0] = '0'; server_smtp_from_address[0] = 'hdipnc@sina.com'; server_smtp_id[0] = '0';server_http_name[0] = 'motionhttp'; server_http_url[0] = ''; server_http_username[0] = ''; server_http_password[0] = ''; server_http_id[0] = '0'; server_http_name[1] = 'exthttp'; server_http_url[1] = ''; server_http_username[1] = ''; server_http_password[1] = ''; server_http_id[1] = '1';  var system_deviceinformation_model = 'MS-C3351';

Example: Get motion window configuration

http://192.168.0.200/cgi-bin/operator/operator.cgi?action=get.event.server&format=inf

Return: No motion detection configuration

HTTP/1.1 200 OK

Cache-Control: no-store, no-cache, must-revalidate, post-check=0, pre-check=0, max-age=-1

Content-Type: text/plain; charset=UTF-8

Transfer-Encoding: chunked

Date: Sat, 01 Jan 2000 20:37:45 GMT

Server: Boa/0.94.13

[EVENT_SERVER]

server_ftp_id_0=0

server_ftp_name_0=msftp0

server_ftp_address_0=192.168.5.1

server_ftp_path_0=default_folder

server_ftp_username_0=ftpuser

server_ftp_password_0=9999

server_smtp_name_0=mssmtp0

server_smtp_address_0=smtp.sina.com

server_smtp_port_0=25

server_smtp_username_0=hdipnc

server_smtp_password_0=hdipnc2013ms

server_smtp_ssl_0=0

server_smtp_from_address_0=hdipnc@sina.com

server_smtp_id_0=0

server_http_name_0=motionhttp

server_http_url_0=

server_http_username_0=

server_http_password_0=

server_http_id_0=0

server_http_name_1=exthttp

server_http_url_1=

server_http_username_1=

server_http_password_1=

server_http_id_1=1

var system_deviceinformation_model=MS-C3351

Return: with event server

HTTP/1.1 200 OK

Cache-Control: no-store, no-cache, must-revalidate, post-check=0, pre-check=0, max-age=-1

Content-Type: text/plain; charset=UTF-8

Transfer-Encoding: chunked

Date: Sat, 01 Jan 2000 21:00:27 GMT

Server: Boa/0.94.13

[EVENT_SERVER]

server_ftp_id_0=0

server_ftp_name_0=msftp0

server_ftp_address_0=192.168.5.1

server_ftp_path_0=default_folder

server_ftp_username_0=ftpuser

server_ftp_password_0=9999

server_smtp_name_0=mssmtp0

server_smtp_address_0=smtp.sina.com

server_smtp_port_0=25

server_smtp_username_0=hdipnc

server_smtp_password_0=hdipnc2013ms

server_smtp_ssl_0=0

server_smtp_from_address_0=hdipnc@sina.com

server_smtp_id_0=0

server_http_name_0=motionhttp

server_http_url_0=http://192.168.9.21/motionalarm

server_http_username_0=admin

server_http_password_0=admin

server_http_id_0=0

server_http_name_1=exthttp

server_http_url_1=http://192.168.9.21/extalarm

server_http_username_1=admin

server_http_password_1=admin

server_http_id_1=1

var system_deviceinformation_model=MS-C3351

- 2.8.2 update.event.ftp.server

Update ftp server for event rule

Example: Update FTP server

http://192.168.0.200/cgi-bin/operator/operator.cgi?action=update.event.ftp.server&server_ftp_id=1&server_ftp_name=FTP1&server_ftp_address=192.168.0.100&server_ftp_port=21&server_ftp_path=%2Ftmp1&server_ftp_username=test&server_ftp_password=test

- 2.8.3 add.event.ftp.server

Add ftp server for event rule

Note : The server_ftp_id will be created by system automaticly.

Example: Add FTP server

http://192.168.0.200/cgi-bin/operator/operator.cgi?action=add.event.ftp.server&server_ftp_id=&server_ftp_name=FTP1&server_ftp_address=192.168.0.100&server_ftp_port=21&server_ftp_path=%2Ftmp&server_ftp_username=test&server_ftp_password=test

- 2.8.4 del.event.ftp.server

Del ftp server by ID

| Parameter | Values | Description |
| --- | --- | --- |
| server_ftp_id | An unsigned integer | FTP server ID |

Example: Delete FTP server

http://192.168.0.200/cgi-bin/operator/operator.cgi?action=del.event.ftp.server&server_ftp_id=1

- 2.8.5 update.event.http.server

Update http server for event rule

Example: Update http server

http://192.168.0.200/cgi-bin/operator/operator.cgi?action=update.event.http.server&server_http_id=1&server_http_url=http%3A%2F%2F192.168.5.100%2Fnotify.cgi&server_http_username=test&server_http_password=test

- 2.8.6 add.event.http.server

Add http server for event rule

Note : The server_http_id will be created by system automatically.

Example: Add http server

http://192.168.0.200/cgi-bin/operator/operator.cgi?action=add.event.http.server&server_http_id=&server_http_url=http%3A%2F%2F192.168.5.100%2Fnotify.cgi&server_http_username=test&server_http_password=test

- 2.8.7 del.event.http.server

Delete http server by ID

| Parameter | Values | Description |
| --- | --- | --- |
| server_http_id | integer | HTTP server ID |

Delete http server

http://192.168.0.200/cgi-bin/operator/operator.cgi?action=del.event.http.server&server_http_id=1

- 2.8.8 update.event.tcp.server (not implemented)

Update TCP server for event rule

Example: Update TCP server

http://192.168.0.200/cgi-bin/operator/operator.cgi?action=update.event.tcp.server&server_tcp_id=1&server_tcp_name=TCP1&server_tcp_address=192.168.0.100&server_tcp_port=12346

- 2.8.9 add.event.tcp.server (not implemented)

Add TCP server for event rule

http://<serverIP>/cgi-bin/operator/operator.cgi?action=add.event.tcp.server[&<parameter>=<value>…]

Note : The server_tcp_id will be created by system automaticly.

Example: Add TCP server

http://192.168.0.200/cgi-bin/operator/operator.cgi?action=add.event.tcp.server&server_tcp_id=&server_tcp_name=TCP1&server_tcp_address=192.168.0.100&server_tcp_port=12345

- 2.8.10 del.event.tcp.server (not implemented)

Delete TCP server by ID

| Parameter | Values | Description |
| --- | --- | --- |
| server_tcp_id | integer | TCP server ID |

Example: Delete TCP server

http://192.168.0.200/cgi-bin/operator/operator.cgi?action=del.event.tcp.server&server_tcp_id=1

- 2.8.11 update.event.smtp.server

Update SMTP server for event rule

Example: Update SMTP server configuration

http://192.168.0.200/cgi-bin/operator/operator.cgi?action=update.event.smtp.server&server_smtp_id=1&server_smtp_name=SMTP&server_smtp_from_address=test%40gmail.com&server_smtp_address=smtp.gmail.com&server_smtp_port=25&server_smtp_ssl=1&server_smtp_username=test&server_smtp_password=test

- 2.8.12 add.event.smtp.server

Add SMTP server for event rule

Note : The server_smtp_id will be created by system automaticly.

Example: Add SMTP server

http://192.168.0.200/cgi-bin/operator/operator.cgi?action=add.event.smtp.server&server_smtp_id=&server_smtp_name=SMTP&server_smtp_from_address=test%40gmail.com&server_smtp_address=smtp.gmail.com&server_smtp_port=25&server_smtp_ssl=1&server_smtp_username=test&server_smtp_password=test

- 2.8.13 del.event.smtp.server

Syntax: Delete SMTP server by ID

| Parameter | Values | Description |
| --- | --- | --- |
| server_smtp_id | integer | SMTP ID, same as get |

Example: Delete SMTP server

http://192.168.0.200/cgi-bin/operator/operator.cgi?action=del.event.smtp.server&server_smtp_id=1

- 2.8.14 check.smtp.email

Test Email server

Use the smtp config for test.

Example: Testing Email setting

http://192.168.0.200/cgi-bin/operator/operator.cgi?action=check.smtp.emai

Return: requested testing success

HTTP/1.1 200 OK

Cache-Control: no-store, no-cache, must-revalidate, post-check=0, pre-check=0, max-age=-1

Content-Type: text/html; charset=UTF-8

Transfer-Encoding: chunked

Date: Sat, 01 Jan 2000 20:45:56 GMT

Server: Boa/0.94.13

Return: requested testing failed

HTTP/1.1 200 OK

Cache-Control: no-store, no-cache, must-revalidate, post-check=0, pre-check=0, max-age=-1

Content-Type: text/html; charset=UTF-8

Transfer-Encoding: chunked

Date: Sat, 01 Jan 2000 20:45:25 GMT

Server: Boa/0.94.13

email: FATAL: Smtp error: 334 VXNlcm5hbWU6

334 UGFzc3dvcmQ6

535-5.7.1 Username and Password not accepted. Learn more at

535 5.7.1 http://mail.google.com/support/bin/answer.py?answer=14257 t2sm1056965ybb.29

- 2.9 EVENT SERVER I/O

- 2.9.1 get.event.io

Set I/O configuration for event rule

| Parameter | Values | Description |
| --- | --- | --- |
| system_misc_powerled(not implemented) | Boolean | Enable power LED 0 : No 1 : Yes |
| io_relayoutput_io_type_<ID> | Boolean | I/O Type 0 : GPOUT 1 : GPIN |
| io_relayoutput_token_<ID> | An unsigned integer | Unique identifier referencing the physical entity of same type. |
| io_relayoutput_properties_mode_<ID>(not implemented) | Boolean | 0 : Monostable 1 : Bistable Bistable – After setting the state, the relay remains in this state. Monostable – After setting the state, the relay returns to its idle state after the specified time. |
| io_relayoutput_properties_delaytime_<ID>(not implemented) | An unsigned integer 1~200 | DelayTime Time after which the relay returns to its idle state if it is in monostable mode. If the Mode field is set to bistable mode the value of the parameter can be ignored. |
| io_relayoutput_properties_idlestate_<ID> | Boolean | Idle state 0 : open 1 : grounded 'open' means that the relay is open when the relay state is set to 'inactive' through the trigger command and closed when the state is set to 'active' through the same command. 'grounded' means that the relay is closed when the relay state is set to 'inactive' through the trigger command and open when the state is set to 'active' through the same command. |
| io_relayoutput_logicalstate_<ID> | Boolean | Logical state 0 : active 1 : inactive |

| Parameter | Values | Description |
| --- | --- | --- |
| format | inf | Export INF format |

Example: request I/O configuration for event rule

http://192.168.0.200/cgi-bin/operator/operator.cgi?action=get.event.io

Return: requested I/O configuration for event rule

HTTP/1.1 200 OK

Cache-Control: no-store, no-cache, must-revalidate, post-check=0, pre-check=0, max-age=-1

Content-Type: text/html; charset=UTF-8

Transfer-Encoding: chunked

Date: Sat, 01 Jan 2000 21:13:58 GMT

Server: Boa/0.94.13

var system_misc_powerled = '1';var model_hardware_serialport = '1'; var model_hardware_gpin = '1'; var model_hardware_gpout = '1'; var model_hardware_powerled = '1';io_relayoutput_token[0] = '1'; io_relayoutput_properties_mode[0] = '0'; io_relayoutput_properties_delaytime[0] = 5; io_relayoutput_properties_idlestate[0] = '1'; io_relayoutput_logicalstate[0] = '1'; io_relayoutput_io_type[0] = '1';io_relayoutput_token[1] = '1'; io_relayoutput_properties_mode[1] = '0'; io_relayoutput_properties_delaytime[1] = 5; io_relayoutput_properties_idlestate[1] = '1'; io_relayoutput_logicalstate[1] = '1'; io_relayoutput_io_type[1] = '0';

Example: request I/O configuration for event rule

http://192.168.0.200/cgi-bin/operator/operator.cgi?action=get.event.io&format=inf

Return: requested I/O configuration for event rule

HTTP/1.1 200 OK

Cache-Control: no-store, no-cache, must-revalidate, post-check=0, pre-check=0, max-age=-1

Content-Type: text/plain; charset=UTF-8

Transfer-Encoding: chunked

Date: Sat, 01 Jan 2000 21:14:01 GMT

Server: Boa/0.94.13

[EVENT_IO]

system_deviceinformation_model=AM4210

system_misc_powerled=1

model_hardware_serialport=1

model_hardware_gpin=1

model_hardware_gpout=1

model_hardware_powerled=1

io_relayoutput_token_0=1

io_relayoutput_properties_mode_0=0

io_relayoutput_properties_delaytime_0=5

io_relayoutput_properties_idlestate_0=1

io_relayoutput_logicalstate_0=1

io_relayoutput_io_type_0=1

io_relayoutput_token_1=1

io_relayoutput_properties_mode_1=0

io_relayoutput_properties_delaytime_1=5

io_relayoutput_properties_idlestate_1=1

io_relayoutput_logicalstate_1=1

io_relayoutput_io_type_1=0

2.9.2 set.event.io

Set I/O configuration for event rule

Example: request PIR event configuration

http://192.168.0.200/cgi-bin/operator/operator.cgi?action=set.event.io&system_misc_powerled=1&io_relayoutput_token_0=1&io_relayoutput_properties_mode_0=0&io_relayoutput_properties_idlestate_0=1&io_relayoutput_properties_delaytime_0=5&io_relayoutput_logicalstate_0=1&io_relayoutput_io_type_0=1&io_relayoutput_token_1=1&io_relayoutput_properties_mode_1=0&io_relayoutput_properties_idlestate_1=1&io_relayoutput_properties_delaytime_1=5&io_relayoutput_logicalstate_1=1&io_relayoutput_io_type_1=0

- 2.9.3 active.event.io (not implemented)

Syntax: Trigger GPOUT

| Parameter | Values | Description |
| --- | --- | --- |
| io_command | <token>,<active> | Token: 0~5 GPOUT number, max numbers depends on model type. Active: 0~1 Status : 0 : Inactive ( equals to normal status ) 1 : Active ( the reverse of normal status ) |

Example: Tigger GPOUT #0 to active

http://192.168.0.200/cgi-bin/operator/operator.cgi?action=active.event.io&io_command=0,1

- 2.10 EVENT PIR (not implemented)

- 2.10.1 get.event.pir (not implemented)

Get PIR event configuration

| Parameter | Values | Description |
| --- | --- | --- |
| event_rule_name | String, max 20 | The alias of an event rule |
| event_rule_enabled | Boolean | Enable the event rule 0 : No 1 : Yes |
| event_rule_periodic | Boolean | Event rule type 0 : Always 1 : Schedule |
| event_rule_weekdays | String, max 6 | Format : xxxxxx 100000 : Sunday is set 100001 : Sunday and Saturday are set. |
| event_rule_starttime | An unsigned integer 0~3600x24 | When periodic is 1, the start time. |
| event_rule_duration | An unsigned integer 0~3600x24 | When periodic is 1, the end time. |
| event_rule_triggerbymotion_enable | Boolean | Enable Trigger by motion detection. 0 : No 1 : Yes |
| event_rule_triggerbymotion_window_id | An unsigned integer | Motion detection windows ID. |
| event_rule_triggerbyboot_enable | Boolean | Enable trigger when device boots. 0 : No 1 : Yes |
| event_rule_triggerbyschedule_enable | Boolean | Enable trigger by schedule. 0 : No 1 : Yes |
| event_rule_actionupload_enabled | Boolean | Enable upload action. 0 : No 1 : Yes |
| event_rule_actionupload_type | 0 | Fixed value. ( Upload via FTP ) |
| event_rule_actionupload_ftp_id | An unsigned integer | FTP server id. |
| event_rule_actionupload_file_type | 0 | Fixed value. ( JPEG ) |
| event_rule_actionupload_post_alarm | An unsigned integer 1~200 | When trigger is not by ‘Schedule’. |
| event_rule_actionupload_interval | An unsigned integer 1~300000 | 1.1 seconds = 1100. |
| event_rule_actionemail_enabled | Boolean | Enable email action. 0 : No 1 : Yes |
| event_rule_actionemail_smtp_id | An unsigned integer | SMTP server id. |
| event_rule_actionemail_file_type |  |  |
| event_rule_actionemail_post_alarm | An unsigned integer 1~200 | When trigger is not by ‘Schedule’. |
| event_rule_actionemail_interval | An unsigned integer 1~300000 | 1.1 seconds = 1100. |
| event_rule_actionemail_smtp_email | String, max 80 | Target email address. |
| event_rule_actionemail_smtp_subject | String, max 80 | Email subject. |
| event_rule_actionemail_smtp_information | String, max 80 | Email message. |
| event_rule_actionemail_smtp_upload | Boolean | Enable upload file. 0 : No 1 : Yes |
| event_rule_actionhttp_enabled | Boolean | Enable HTTP action. 0 : No 1 : Yes |
| event_rule_actionhttp_http_id | An unsigned integer | HTTP server id. |
| event_rule_actionhttp_http_post_alarm | An unsigned integer 1~200 | When trigger is not by ‘Schedule’. |
| event_rule_actionhttp_http_interval | An unsigned integer 1~300000 | 1.1 seconds = 1100. |
| event_rule_actionhttp_http_message | String, max 80 | HTTP message. |
| event_rule_actiontcp_enabled | Boolean | Enable TCP action. |
| event_rule_actiontcp_tcp_id | An unsigned integer | TCP server id. |
| event_rule_actiontcp_tcp_post_alarm | An unsigned integer 1~200 | When trigger is not by ‘Schedule’. |
| event_rule_actiontcp_tcp_interval | An unsigned integer 1~300000 | 1.1 seconds = 1100. |
| event_rule_actiontcp_tcp_message | String, max 80 | TCP message. |
| event_rule_actionrecord_enabled | Boolean | Enable record to SD. 0 : No 1 : Yes |
| event_rule_triggerbygpin_enable | Boolean | 0 : No 1 : Yes |
| event_rule_triggerbygpin_id | An unsigned integer | GPIN id. |
| event_rule_actiongpout_enabled | Boolean | 0 : No 1 : Yes |
| event_rule_actiongpout_id | An unsigned integer | GPOUT id. |
| event_rule_id | An unsigned integer | The rule index id. |

| Parameter | Values | Description |
| --- | --- | --- |
| format | inf | Export INF format |

Example: request PIR event configuration

http://192.168.0.200/cgi-bin/operator/operator.cgi?action=get.event.pir

Return:

HTTP/1.1 200 OK

Cache-Control: no-store, no-cache, must-revalidate, post-check=0, pre-check=0, max-age=-1

Content-Type: text/html; charset=UTF-8

Transfer-Encoding: chunked

Date: Sat, 01 Jan 2000 23:34:23 GMT

Server: Boa/0.94.13

var system_io = 1;var  model_hardware_gpout = '1';var event_rule_enabled = '0'; var event_rule_actionupload_enabled = '0'; var event_rule_actionemail_enabled = '0'; var event_rule_actionhttp_enabled = '0'; var event_rule_actiontcp_enabled = '132'; var event_rule_actionrecord_enabled = '0'; var event_rule_actiongpout_enabled = '0'; var event_rule_actionupload_type = '0'; var event_rule_actionupload_ftp_id = '0'; var event_rule_actionupload_interval = '1000'; var event_rule_actionupload_file_type = '0'; var event_rule_actionemail_smtp_id = '0'; var event_rule_actionemail_smtp_email = ''; var event_rule_actionemail_smtp_subject = ''; var event_rule_actionemail_smtp_information = ''; var event_rule_actionemail_smtp_upload = '0'; var event_rule_actionemail_interval = '2000'; var event_rule_actionemail_file_type = '0'; var event_rule_actionhttp_http_id = '0'; var event_rule_actionhttp_http_interval = '1000'; var event_rule_actionhttp_http_parameter = ''; var event_rule_actionhttp_http_message = ''; var event_rule_actiontcp_tcp_id = '0';var event_rule_actiontcp_tcp_interval = '1000'; var event_rule_actiontcp_tcp_message = ''; var event_rule_triggerbygpin_id = '0'; var event_rule_actiongpout_id = '2'; var event_rule_actionupload_post_alarm = '10'; var event_rule_actionemail_post_alarm = '10'; var event_rule_actionhttp_http_post_alarm = '10'; var event_rule_actiontcp_tcp_post_alarm = '10';var isRecord = '0'; io_relayoutput_token[0] = '1'; io_relayoutput_properties_mode[0] = '0'; io_relayoutput_properties_delaytime[0] = 5; io_relayoutput_properties_idlestate[0] = '1'; io_relayoutput_logicalstate[0] = '1'; io_relayoutput_io_type[0] = '1';io_relayoutput_token[1] = '1'; io_relayoutput_properties_mode[1] = '0'; io_relayoutput_properties_delaytime[1] = 5; io_relayoutput_properties_idlestate[1] = '1'; io_relayoutput_logicalstate[1] = '1'; io_relayoutput_io_type[1] = '0';

Example: request PIR event configuration

http://192.168.0.200/cgi-bin/operator/operator.cgi?action=get.event.pir&format=inf

Return:

HTTP/1.1 200 OK

Cache-Control: no-store, no-cache, must-revalidate, post-check=0, pre-check=0, max-age=-1

Content-Type: text/plain; charset=UTF-8

Transfer-Encoding: chunked

Date: Sat, 01 Jan 2000 23:34:29 GMT

Server: Boa/0.94.13

[EVENT_PIR]

system_io=1

model_hardware_gpout=1

event_rule_enabled=0

event_rule_actionupload_enabled=0

event_rule_actionemail_enabled=0

event_rule_actionhttp_enabled=0

event_rule_actiontcp_enabled=132

event_rule_actionrecord_enabled=0

event_rule_actiongpout_enabled=0

event_rule_actionupload_type=0

event_rule_actionupload_ftp_id=0

event_rule_actionupload_interval=1000

event_rule_actionupload_file_type=0

event_rule_actionemail_smtp_id=0

event_rule_actionemail_smtp_email=

event_rule_actionemail_smtp_subject=

event_rule_actionemail_smtp_information=

event_rule_actionemail_smtp_upload=0

event_rule_actionemail_interval=2000

event_rule_actionemail_file_type=0

event_rule_actionhttp_http_id=0

event_rule_actionhttp_http_interval=1000

event_rule_actionhttp_http_parameter=

event_rule_actionhttp_http_message=

event_rule_actiontcp_tcp_id=0

event_rule_actiontcp_tcp_interval=1000

event_rule_actiontcp_tcp_message=

event_rule_triggerbygpin_id=0

event_rule_actiongpout_id=2

event_rule_actionupload_post_alarm=10

event_rule_actionemail_post_alarm=10

event_rule_actionhttp_http_post_alarm=10

event_rule_actiontcp_tcp_post_alarm=10

isRecord=0

io_relayoutput_token_0=1

io_relayoutput_properties_mode_0=0

io_relayoutput_properties_delaytime_0=5

io_relayoutput_properties_idlestate_0=1

io_relayoutput_logicalstate_0=1

io_relayoutput_io_type_0=1

io_relayoutput_token_1=1

io_relayoutput_properties_mode_1=0

io_relayoutput_properties_delaytime_1=5

io_relayoutput_properties_idlestate_1=1

io_relayoutput_logicalstate_1=1

io_relayoutput_io_type_1=0

- 2.10.2 set.event.pir (not implemented)

Set PIR configuration for event rule

Example: Set PIR event configuration

http://192.168.0.200/cgi-bin/operator/operator.cgi?action=set.event.pir&event_rule_enabled=0&event_rule_actionupload_enabled=0&event_rule_actionupload_ftp_id=0&event_rule_actionupload_interval=1000&event_rule_actionemail_enabled=0&event_rule_actionemail_smtp_id=0&event_rule_actionemail_interval=1000&event_rule_actionemail_smtp_email=&event_rule_actionemail_smtp_subject=&event_rule_actionemail_smtp_information=&event_rule_actionemail_smtp_upload=0&event_rule_actionhttp_enabled=0&event_rule_actionhttp_http_id=0&event_rule_actionhttp_http_interval=1000&event_rule_actionhttp_http_parameter=&event_rule_actionhttp_http_message=&event_rule_actiontcp_enabled=0&event_rule_actiontcp_tcp_id=0&event_rule_actiontcp_tcp_interval=1000&event_rule_actiontcp_tcp_message=&event_rule_actionrecord_enabled=0&event_rule_actiongpout_enabled=0&event_rule_actiongpout_id=2&event_rule_actionupload_post_alarm=10&event_rule_actionemail_post_alarm=10&event_rule_actionhttp_http_post_alarm=10&event_rule_actiontcp_tcp_post_alarm=10

- 2.11 YOICS (not implemented)

- 2.11.1 get.yoics.stat (not implemented)

Get YOICS service state

| Parameter | Values | Description |
| --- | --- | --- |
| format | inf | Export INF format |

Example: request YOICS service state

http://192.168.0.200/cgi-bin/operator/operator.cgi?action=get.yoics.stat

Return:

HTTP/1.1 200 OK

Cache-Control: no-store, no-cache, must-revalidate, post-check=0, pre-check=0, max-age=-1

Content-Type: text/html; charset=UTF-8

Transfer-Encoding: chunked

Date: Sat, 01 Jan 2000 23:44:59 GMT

Server: Boa/0.94.13

var system_misc_yoics_enable = '1';

Example: request YOICS service state

http://192.168.0.200/cgi-bin/operator/operator.cgi?action=get.yoics.stat&format =inf

Return:

HTTP/1.1 200 OK

Cache-Control: no-store, no-cache, must-revalidate, post-check=0, pre-check=0, max-age=-1

Content-Type: text/plain; charset=UTF-8

Transfer-Encoding: chunked

Date: Sat, 01 Jan 2000 23:45:04 GMT

Server: Boa/0.94.13

[YOICS_STATUS]

system_misc_yoics_enable=1

- 2.11.2 set.yoics.stat (not implemented)

Syntax: Set YOICS service state

| Parameter | Values | Description |
| --- | --- | --- |
| system_misc_yoics_enable | Boolean | Enable YOICS serice 0 : No 1 : Yes |

Example: Enable YOICS service

http://192.168.0.200/cgi-bin/operator/operator.cgi?action=set.yoics.stat&system_misc_yoics_enable=1

- 3. viewer.cgi

Permissions: Admin, Operator, Viewer

Usage:

http://<URL>/cgi-bin/viewer/viewer.cgi?action=value>[&<parameter=value>]

Action List:

| Action Type | Action Name | INF | description |
| --- | --- | --- | --- |
| Live view | get.liveview | V | Get live view information |
|  | rotate.dptz |  | Set DPTZ |
| PTZ | ptz.control |  | Control a PTZ. |
|  | ptz.speed |  | Set PTZ speed |
| System | get.setup | V |  |
|  | get.Model | V |  |

- 3.1 Live view

- 3.1.1 get.liveview

Syntax: Get live view information

| Parameter | Values | Description |
| --- | --- | --- |
| format | inf | Export INF format |

Example: request live view information

http://192.168.0.200/cgi-bin/viewer/viewer.cgi?action=get.liveview

Return:

HTTP/1.1 200 OK

Cache-Control: no-store, no-cache, must-revalidate, post-check=0, pre-check=0, max-age=-1

Content-Type: text/html; charset=UTF-8

Transfer-Encoding: chunked

Date: Sat, 01 Jan 2000 23:48:34 GMT

Server: Boa/0.94.13

var system_deviceinformation_vender = 'Milesight'; var system_deviceinformation_sensor = '3m_0331'; var system_deviceinformation_tvstandards = '-1'; var system_deviceinformation_model  = 'MS-C3351'; var rtsp_port = '554'; var model_hardware_serialport = '1'; var ptz_setting_enable = '1'; var ptz_setting_protocol = '1'; var ptz_setting_speed = '5'; media_profile_id_0 = '0'; media_profile_url_0 = 'main'; media_profile_dptz_enabled_0 = '0'; media_profile_resolution_width_0 = '1920'; media_profile_resolution_height_0 = '1080'; media_profile_framerate_0 = '1'; media_profile_id[1] = '1'; media_profile_url[1] = 'sub'; media_profile_resolution_width[1] = '320'; media_profile_resolution_height[1] = '192'; media_profile_framerate[1] = '1'; var islogo  = '0';

Example: request live view information

http://192.168.0.200/cgi-bin/viewer/viewer.cgi?action=get.liveview&format=inf

Return:

HTTP/1.1 200 OK

Cache-Control: no-store, no-cache, must-revalidate, post-check=0, pre-check=0, max-age=-1

Content-Type: text/plain; charset=UTF-8

Transfer-Encoding: chunked

Date: Sat, 01 Jan 2000 23:48:42 GMT

Server: Boa/0.94.13

[LIVE_VIEW]

system_deviceinformation_vender=Milesight

system_deviceinformation_sensor=3m_0331

system_deviceinformation_tvstandards=-1

system_deviceinformation_model =MS-C3351

rtsp_port=554

model_hardware_serialport=1

ptz_setting_enable=1

ptz_setting_protocol=1

ptz_setting_speed=5

media_profile_id_0=0

media_profile_url_0=main

media_profile_resolution_width_0=1920

media_profile_resolution_height_0=1080

media_profile_framerate_0=1

media_profile_id_1=1

media_profile_url_1=sub

media_profile_resolution_width_1=320

media_profile_resolution_height_1=192

media_profile_framerate_1=1

islogo=0

- 3.1.2 rotate.dptz (not implemented)

Set DPTZ

| Parameter | Values | Description |
| --- | --- | --- |
| media_profile_dptz_horzoffset | integer | DPTZ top left X coordinates |
| media_profile_dptz_vertoffset | integer | DPTZ top left Y coordinates |
| media_profile_id | 2,4 | DPTZ stream ID |
| system_deviceinformation_sensor | 2m, vga, composite | Sensor type |
| system_deviceinformation_tvstandards | 0,1 | Video standard (AM7000 only) 0 NTSC, 1 PAL |

Example: Move DPTZ top left corrdinates to (8, 18) for stream 3

http://192.168.0.200/cgi-bin/viewer/viewer.cgi?action=rotate.dptz&media_profile_id=2&media_profile_dptz_vertoffset=8&media_profile_dptz_horzoffset=18&system_deviceinformation_sensor=2m&system_deviceinformation_tvstandards=0

- 3.2 PTZ control

- 3.2.1 ptz.control

Control a PTZ.

| Parameter | Values | Description |
| --- | --- | --- |
| ptz_cmd | An unsigned integer | PTZ control command 0: STOP 1: UP 2: DOWN 3: LEFT 4. RIGHT 5.TOP RIGHT 6.TOP LEFT 7.BOTTOM RIGHT 8.BOTTOM LEFT 32: ZOOM_IN 33: ZOOM_OUT 34: FOCUS_NEAR 35: FOCUS_FAR 36: IRIS_CLOSE 37: IRIS_OPEN 48: ZOOM_AUTO 49: FOCUS_AUTO 50: IRIS_AUTO 64: Preset Point SET 65: Preset Point CLEAR 66: Preset Point GOTO 80: AUTOPAN_ON 81: AUTOPAN_OFF 82: ZONESCAN_ON 83: ZONESCAN_OFF 128: FLIP_ON 129: FLIP_OFF 130: REMOTE_RESET |
| ptz_go_point | An unsigned integer 1~20 | Go to a preset position according to the preset number. (Only using PTZ command 64/65/66) |
| ptz_status | Boolean | Stop PTZ after PTZ control command 0 : No 1 : Yes |

Example: set the PTZ UP with STOP

http://192.168.0.200/cgi-bin/cgi-bin/viewer/viewer.cgi?action=ptz.control&ptz_cmd=1&ptz_status=1

Example: set the PTZ LIFE without STOP

http://192.168.0.200/cgi-bin/cgi-bin/viewer/viewer.cgi?action=ptz.control&ptz_cmd=3&ptz_status=0

Example: set the PTZ goto preset 10

http://192.168.0.200/cgi-bin/cgi-bin/viewer/viewer.cgi?action=ptz.control&ptz_cmd=66&ptz_go_point=10

- 3.2.2 ptz.speed

Set PTZ speed.

| Parameter | Values | Description |
| --- | --- | --- |
| ptz_setting_speed | 1~10 | PTZ speed |

Example: set the PTZ speed 10

http://192.168.0.200/cgi-bin/cgi-bin/viewer/viewer.cgi?action=ptz.speed&ptz_setting_speed=10

- 3.3 System

- 3.3.1 get.setup

Get misc setting

| Parameter | Values | Description |
| --- | --- | --- |
| format | inf | Export INF format |

Example: request the misc setting

http://192.168.0.200/cgi-bin/viewer/viewer.cgi?action=get.setup

Return:

HTTP/1.1 200 OK

Cache-Control: no-store, no-cache, must-revalidate, post-check=0, pre-check=0, max-age=-1

Content-Type: text/html; charset=UTF-8

Transfer-Encoding: chunked

Date: Sat, 01 Jan 2000 23:56:05 GMT

Server: Boa/0.94.13

var system_deviceinformation_vender = 'Milesight'; var system_misc_haveyoics = '0'; var islogo = '0';

Example: request the misc setting

http://192.168.0.200/cgi-bin/viewer/viewer.cgi?action=get.setup&format=inf

Return:

HTTP/1.1 200 OK

Cache-Control: no-store, no-cache, must-revalidate, post-check=0, pre-check=0, max-age=-1

Content-Type: text/plain; charset=UTF-8

Transfer-Encoding: chunked

Date: Sat, 01 Jan 2000 23:56:11 GMT

Server: Boa/0.94.13

[SETUP]

system_deviceinformation_vender=Milesight

system_misc_haveyoics=0

islogo=0

- 3.3.2 get.Model

Get Model name

| Parameter | Values | Description |
| --- | --- | --- |
| format | inf | Export INF format |

Example: request the model name

http://192.168.0.200/cgi-bin/viewer/viewer.cgi?action=get.Model

Return:

HTTP/1.1 200 OK

Cache-Control: no-store, no-cache, must-revalidate, post-check=0, pre-check=0, max-age=-1

Content-Type: text/html; charset=UTF-8

Transfer-Encoding: chunked

Date: Sat, 01 Jan 2000 23:58:22 GMT

Server: Boa/0.94.13

var system_deviceinformation_vender = 'Milesight'; var name = 'MS-C3351'; var system_deviceinformation_model = 'MS-C3351';

Example: request the model name

http://192.168.0.200/cgi-bin/viewer/viewer.cgi?action=get.Model&format=inf

Return:

HTTP/1.1 200 OK

Cache-Control: no-store, no-cache, must-revalidate, post-check=0, pre-check=0, max-age=-1

Content-Type: text/plain; charset=UTF-8

Transfer-Encoding: chunked

Date: Sat, 01 Jan 2000 23:58:30 GMT

Server: Boa/0.94.13

[MODEL]

brand=Milesight

name=MS-C3351

system_deviceinformation_model=MS-C3351

- 4. Operator misc

Permissions: Admin, Operator

Usage:

http://<URL>/cgi-bin/operator/<CGI>.cgi?action=<value>[&<parameter=value>]

Action List:

| CGI | Action Name | INF | description |
| --- | --- | --- | --- |
| filelist | file.list | V | Get record file list. |
|  | del.files |  | Delete files. |
|  | Playback (not implement) |  | Playback a recordfile. |
| recycle | - | V |  |
| snapshot |  |  | Get snapshot jpeg. |

- 4.1 filelist.cgi

- 4.1.1 file.list

Get file list by search time range

| Parameter | Values | Description |
| --- | --- | --- |
| s_year | An unsigned integer | The year of search start time |
| s_month | An unsigned integer 0~11 | The month of search start time |
| s_day | An unsigned integer 1~31 | The day of search start time |
| s_hour | An unsigned integer 0~23 | The hour of search start time |
| e_year | An unsigned integer | The year of search end time |
| e_month | An unsigned integer 0~11 | The month of search end time |
| e_day | An unsigned integer 1~31 | The day of search end time |
| e_hour | An unsigned integer 0~23 | The hour of search end time |
| page | An unsigned integer | The page of all file list |
| format | inf | Export INF format |

Example: First search

http://192.168.0.200/cgi-bin/operator/filelist.cgi?action=file.list&action=file.list&s_year=2013&s_month=5&s_day=15&s_hour=14&e_year=2013&e_month=5&e_day=26&e_hour=14

Return: Record file list (page 0).

HTTP/1.1 200 OK

Cache-Control: no-store, no-cache, must-revalidate, post-check=0, pre-check=0, max-age=-1

Content-Type: text/html; charset=UTF-8

Transfer-Encoding: chunked

Date: Tue, 21 Jun 2011 14:23:10 GMT

Server: Boa/0.94.13

Files[0] = '20130523013736.avi'; Files[1] = '20130517094723.avi'; Files[2] = '20130517094652.avi'; Files[3] = '20130517094621.avi'; Files[4] = '20130517094550.avi'; Files[5] = '20130517094519.avi'; Files[6] = '20130517094448.avi'; Files[7] = '20130517094417.avi'; Files[8] = '20130517094346.avi'; Files[9] = '20130517094315.avi'; Files[10] = '20130517094245.avi'; Files[11] = '20130517094213.avi'; Files[12] = '20130517094142.avi'; Files[13] = '20130517094112.avi'; Files[14] = '20130517094041.avi'; Files[15] = '20130517094010.avi'; Files[16] = '20130517093938.avi'; Files[17] = '20130517093907.avi'; Files[18] = '20130517093835.avi'; Files[19] = '20130517093739.avi'; var total_files = '243'; var files_count = '20';

Example: First search

http://192.168.5.227:8080/cgi-bin/operator/filelist.cgi?action=file.list&s_year=2013&s_month=5&s_day=15&s_hour=14&e_year=2013&e_month=5&e_day=26&e_hour=14&format=inf

Return: Record file list (page 0).

HTTP/1.1 200 OK

Cache-Control: no-store, no-cache, must-revalidate, post-check=0, pre-check=0, max-age=-1

Content-Type: text/plain; charset=UTF-8

Transfer-Encoding: chunked

Date: Tue, 21 Jun 2011 14:26:46 GMT

Server: Boa/0.94.13

Files0=20130523013736.avi

Files1=20130517094723.avi

Files2=20130517094652.avi

Files3=20130517094621.avi

Files4=20130517094550.avi

Files5=20130517094519.avi

Files6=20130517094448.avi

Files7=20130517094417.avi

Files8=20130517094346.avi

Files9=20130517094315.avi

Files10=20130517094245.avi

Files11=20130517094213.avi

Files12=20130517094142.avi

Files13=20130517094112.avi

Files14=20130517094041.avi

Files15=20130517094010.avi

Files16=20130517093938.avi

Files17=20130517093907.avi

Files18=20130517093835.avi

Files19=20130517093739.avi

total_files=243

files_count=20

- 4.1.2 del.files

Delete record file by name

| Parameter | Values | Description |
| --- | --- | --- |
| files | integer | Total delete file numbers |
| File<NUM> | string | Delete file name |

Example: Delete file

http://192.168.0.200/cgi-bin/operator/filelist.cgi?actions=del.files& files=1&file0=20130517094315.avi

- 4.1.3 playback

Playback record file by name

| Parameter | Values | Description |
| --- | --- | --- |
| Files | string | The file name of play file |

Example: Playback

http://192.168.0.200/cgi-bin/operator/operator.cgi?action=filelist.playback&filename=2000-01-03_014500_CH0-47.file

- 4.1.4 recycle.cgi

Get SD card information

| Parameter | Values | Description |
| --- | --- | --- |
| format | inf | Export INF format |

Example:

http://192.168.0.200/cgi-bin/operator/recycle.cgi

Return:

HTTP/1.1 200 OK

Cache-Control: no-store, no-cache, must-revalidate, post-check=0, pre-check=0, max-age=-1

Content-Type: text/html; charset=UTF-8

Transfer-Encoding: chunked

Date: Sun, 02 Jan 2000 00:26:21 GMT

Server: Boa/0.94.13

var TotalSize='31158272';var UsedSize='2127104'; var FreeSize='29031168';var Ready = '1';

Example:

http://192.168.0.200/cgi-bin/operator/recycle.cgi?format=inf

Return:

HTTP/1.1 200 OK

Cache-Control: no-store, no-cache, must-revalidate, post-check=0, pre-check=0, max-age=-1

Content-Type: text/plain; charset=UTF-8

Transfer-Encoding: chunked

Date: Sun, 02 Jan 2000 00:25:03 GMT

Server: Boa/0.94.13

[RECYCLE]

TotalSize=31158272

UsedSize=2127104

FreeSize=29031168

Ready=1

- 4.1.5 Snapshot

The size of the video source :

Example:

http://192.168.0.200/cgi-bin/operator/snapshot.cgi

320 x 240

Example:

http://192.168.0.200/snap.jpg

- 5. Misc CGI

- 5.1 notify.fcgi

Permission: Admin, Operator, Viewer

The client side should keep receiving notification information from camera. It includes all available events or status as follow table.

The notification information is only generated on while event or status changed. If there is no any changed event or status being reported within 30 second, a special tag: “keep_alive” will be send to the client side.

Usage:

http://<serverIP>/cgi-bin/notify.fcgi

Return:

with the following parameters and values

| Name | Values | Description |
| --- | --- | --- |
| md# | active | event motion detection # is triggered or not. |
| input# | active | event input # is triggered or not. |
| output# | active,inactive | status of output # is on or off |
| recording | active | status is recording |
| pir | active |  |
| lightsensor | active |  |
| boot | active |  |
| ipchang | active |  |

Example:

http://192.168.0.200 /cgi-bin/notify.fcgi

Return:

HTTP/1.1 200 OK

Cache-Control: no-store, no-cache, must-revalidate, post-check=0, pre-check=0, max-age=-1

Content-Type: text/html

Transfer-Encoding: chunked

Date: Thu, 23 Jun 2011 15:39:56 GMT

Server: Boa/0.94.13

keep_alive

md#=active

- 5.1.1 set.notify

To set event parameter.

| Parameter | Values | Description |
| --- | --- | --- |
| manualmotion | integer | Set motion alarm Manually |

Example:

http://192.168.0.200 /cgi-bin/notify.fcgi?action=manualmotion

- 5.2 audio.fcgi (not implemented)

Permission: Admin, Operator, Viewer

Anti-push audio stream.

Start Usage:

HTTP method : GET

http://<serverIP>/cgi-bin/notify.fcgi&cmd=start

Respnose :

Success : OK. Id=xxxx

Fail : error messages.

Stop Usage:

HTTP method : GET

http://<serverIP>/cgi-bin/notify.fcgi&cmd=stop&id=xxxx

Respnose :

Success : OK. Connect Close

Fail : error messages.

Post data Usage:

HTTP method : POST

http://<serverIP>/cgi-bin/notify.fcgi&id=xxxx&data=(audio data)

Respnose :

Success : OK.

Fail : error messages.

Allowed audio stream format : mu-law/mono/sample rate:8K/8-bits